Abstract

Protocol definition for communication between 3S series device and 3S application software

Spectra Embedded

Nisarg Parmar

3S COMMUMNICATION PROTOCOL

Version 2.30

are

Table of Contents

Introduction	6

Legends	7

Protocol Definition	8

Examples:	9

Data handling rules	11

Predefined Macros	12

FP module Error flags	12

FP module Parameter List	12

Physical Address For Inputs	13

Physical Address For Outputs	15

Software Event Codes	16

Response Code List	17

Function Code (With Payloads)	19

Get Status (01h)	19

Set Date & Time (02h)	20

Add User (Time Zone Based ACCESS) (03h)	21

Add user (Exclusive Gate wise Access) (04h)	22

Upload File Header (05h)	24

Upload File Data (06h)	25

Delete Particular User (07h)	25

Delete All Users (I.E. DB delete all) (08h)	26

Add User Exception (09h)	26

Delete User Exception (0Ah)	28

Scan Live Finger (0Bh)	28

Set finger Template (0Ch)	29

Delete Finger Template (0Dh)	30

Delete All Finger (0Eh)	31

Read / Write FP module Parameters (0Fh)	31

Sync Time Zones (10h)	32

Add Time Zone Access Group (11h)	32

Delete All Time Zone Groups (12h)	33

Sync Week Off Groups (13h)	34

Set Holiday Group (14h)	35

Delete All Holiday Groups (15h)	35

Controller Setup (16h)	36

Reader Setup (17h)	36

Door Setup (18h)	37

Door Mode Exceptions (19h)	39

Door State Exceptions (1Ah)	41

Area Setup (1Bh)	43

Input Definitions (1Ch)	43

Input to Output Triggers (1Dh)	45

Event To Output Triggers (1Eh)	46

Buzzer LED Setup (1Fh)	47

Display & Brightness Parameters (20h)	48

Sync Card ID Format (21h)	50

Offline Firmware Update (Read Firmware Version) (22h)	51

Offline Firmware Update (Upload Firmware Info) (23h)	52

Offline Firmware Update (Upload Firmware Data) (24h)	52

Offline Firmware Update (Verify & Start upgrade) (25h)	53

Factory Reset (26h)	53

Soft Reset (Restart Device) (27h)	54

Over Ride Outputs (28h)	54

Mi-fare Scan CSN (29h)	55

Mi-fare Write Card (2Ah)	56

Mi-fare Release CSN (2Bh)	56

Mi-fare Sector Mode Config (2Ch)	57

Note For Command (2Ch) :-	58

Sync Reason (2Dh)	60

Pause / Resume Transactions (2Eh)	62

Start Transaction Recovery (2Fh)	62

Clear Transactions (30h)	63

Delete All Rules / Specific Rule(31h)	63

Reset Rule State (32h)	64

Master Slave Setup (33h)	65

Rule Setup (Normal / Simple APB) (34h)	65

Rule Setup (Timed APB) (35h)	66

Rule Setup (Conditional APB) (36h)	67

Rule Setup (Concentric Circle) (37h)	68

Rule Setup (Matrix APB) (38h)	68

Rule Setup (Access Limit) (39h)	70

Rule Setup (Absent User) (3Ah)	70

Rule Setup (Custodian) (3Bh)	71

Rule Setup (Multiple Occupancy) (3Ch)	73

Rule Setup (Sentry Check) (3Dh)	74

Rule Setup (Web Post) (3Eh)	76

Rule Setup (Interlocking) (3Fh)	77

Rule Setup (Group Authentication) (40h)	79

Rule Setup (Multiple Authentication) (40h) (Deprecated, Do not use)	82

Get finger Template (41h)	82

Delete Specific Parameters (42h)	83

User Anti pass-back Override (43h)	84

User Authentication change / Finger Bypass (44h)	85

User Custom Message (45h)	86

NTP Parameters (46h)	87

Camera capture on software events (47h) (variable length command)	88

Device Password Reset (48h)	88

BLE Para (49h)	89

Set Finger Encryption (4Ah)	90

Add Extended User Information (4Bh)	90

Enable / Disable Ethernet Modules (F9h)	92

Sound & Volume Parameters (4Ch)	92

User password (Pin) Change (4Dh)	93

Rule Setup (Escort) (4Eh)	93

Rule Setup (MQTT Publisher) (4Fh)	95

Temperature Detection parameters (50h)	97

Arm disarm mode settings (51h)	98

Mifare MADConfig (52h)	98

Mifare MAD Parameters(52h)	99

OSDP Controller Setup(53h)	100

Osdp reader sETUP(54h)	102

EAC Reader setup(55h)	103

Rule Setup Server Auth(56h)	104

Event Action Extended(57h)	105

Input Action Extended(58h)	105

Confifurable output control (59h)	107

Server Auth Query(D1h)	108

Server Auth Response(D1h)	108

Canteen Management System Commands	110

Item Database (C0h)	110

Item Delete (C1h)	111

Add Item Group(C2h)	111

Group Item Delete (C3h)	112

Vendor DB (C4h)	113

Print Slip Configuration (C5h)	113

Canteen Parameters (C6h)	114

Print Header/Index Data (C7h)	115

Query to Master (C8h) , Master Slave Communication	116

Response from Master (C8h) , Master Slave Communication	117

Canteen Master/Slave Communication Error Codes	119

Push Command Codes (Device To Service)	120

Connection Identifier(E0)	120

Push Transactions (E1h)	120

Push Photo Header (E2h) (Deprecated)	122

Push Photo Data (E3h)	123

Push Hard Alarms (E4h)	124

Push Soft Alarms (E5h)	125

Push GPS Data (E6h)	126

Push Canteen Data (E7h)	126

Recovery Transaction Frame (E8h)	128

Recovery Report Frame (E9h)	129

Get Pending Event list (EAh)	130

New Transaction Frame (EBh)	132

Push Log (ECh)	133

Query commands	135

General Query to Server (D0h)	135

Server Authentication Query / Response (D1h)	135

Change Log	138

Memory_management	142

# Introduction

# Legends

- XXh: any one-byte number in hexadecimal format.

- XXd: any one-byte number in decimal format.

- XXXXXXXXb: any one-byte number in binary format.

- XXYYh: any two-byte number (always in hexadecimal format).

- AABBCCDDh: any four-byte number (always in hexadecimal format).

- CID: Controller ID.

- UID: Unique Identification Number. (I.e. Card ID).

- EXTD ID: Extended Card ID (Currently not in use, can be use to incorporate 7 byte UID)

- SC: Site code of user.

- RFU: Reserved for future use. Should be always zero.

- FC: Function Code

- LRC: Longitudinal Redundancy Check. Algorithm for LRC is stated as below…

- LSB: Lower Significant Byte.

- HSB: Higher Significant byte.

- USB: upper significant byte.

- MSB: Most significant byte.

- R/W: Read or write flag.

- TZ: Time Zone

- APB: Anti Pass Back

# Protocol Definition

Commands are sent to and are received from Device must follow this sequence.

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh where (YYd = (n+1) % 100d) |
| 2 | Total Length (HSB) | XXh where (XXd = (n+1) / 100d) |
| 3 | Command Or Response | 01h : command (use this) 02h : response Any other value invalid |
| 4 | RFU | 01h (fixed value) |
| 5 | Read Write Flag | 00h : Read 01h : Write |
| 6 | FC (Function Code) | Predefined Function Number |
| 7 | Response Required? | 00h: Required 01h: Not required |
| 8 | Payload Length (LSB) | YYh where (YYd = (n-12) % 100d) |
| 9 | Payload Length (HSB) | XXh where (XXd = (n-12) / 100d) |
| 10 to n-2 | Payload | useful data in this frame |
| n - 1 | LRC | LRC of 0 to n-2 |
| n | Footer | Fixed BBh |

Response to above commands should follow this sequence (in case of no payload is there)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | 0Ch (12d) |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h : command (use this) 02h : response Any other value invalid |
| 4 | RFU | 01h (fixed value) |
| 5 | Read Write Flag | 00h : Read 01h : Write |
| 6 | FC (Function Code) | Commanded Function Code |
| 7 | Response Code | XXh |
| 8 | Payload Length (LSB) | 00h |
| 9 | Payload Length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

Response to above commands should follow this sequence (with payload)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh where (YYd = (n+1) % 100d) |
| 2 | Total Length (HSB) | XXh where (XXd = (n+1) / 100d) |
| 3 | Command Or Response | 01h : command (use this) 02h : response Any other value invalid |
| 4 | RFU | 01h (fixed value) |
| 5 | Read Write Flag | 00h : Read 01h : Write |
| 6 | FC (Function Code) | Commanded Function Code |
| 7 | Response Code | XXh |
| 8 | Payload Length (LSB) | YYh (where YYd = length of Payload % 100d) |
| 9 | Payload Length (HSB) | XXh (where XXd = length of Payload / 100d) |
| 9 to n - 2 | Payload | Useful Data |
| n - 1 | LRC |  |
| n | Footer | Fixed BBh |

## Examples:

Get Status Command (FC: 01h) (read flag)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | 0Ch (12d) |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h : command (use this) |
| 4 | RFU | 01h (fixed value) |
| 5 | Read Write Flag | 00h : Read |
| 6 | FC (Function Code) | 01h : get status command |
| 7 | Response Required? | 00h (required) |
| 8 | Payload Length (LSB) | 00h |
| 9 | Payload Length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

Get Status Command Response (response to read flag) (with payload)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 1Bh (27d) |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 02h: Response |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 08h |
| 7 | Response | 01h: Success |
| 8 | Payload length (LSB) | 0Fh |
| 9 | Payload length (HSB) | 00h |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | IPv4 address segment 1 | C0h (if ip is 192.100.100.90) |
| 17 | IPv4 address segment 2 | 64h |
| 18 | IPv4 address segment 3 | 64h |
| 19 | IPv4 address segment 4 | 5Ah |
| 20 | CID LSB | YYh (if CID is XXYYh) |
| 21 | CID HSB | XXh |
| 22 | Year | 00h to 63h |
| 23 | Month | 01h to 0Ch |
| 24 | Date | 01h to 1Fh |
| 25 | Day | 01h to 7h |
| 26 | Hour | 00h to 17h |
| 27 | Minute | 00h to 3Bh |
| 28 | Second | 00h to 3Bh |
| 29 | Communication Interface | 1h: ETH 2h: WIFI 3h: GSM |
| 30 | RFU | 0h |
| 31 | RFU | 0h |
| 32 | Total Users (LSB) | DDh (four byte value AABBCCDDh) |
| 33 | Total Users (HSB) | CCh |
| 34 | Total Users (USB) | BBh |
| 35 | Total Users (MSB) | AAh |
| 36 | Total Trans. (LSB) | DDh (four byte value AABBCCDDh) |
| 37 | Total Trans. (HSB) | CCh |
| 38 | Total Trans. (USB) | BBh |
| 39 | Total Trans. (MSB) | AAh |
| 40 | Free Scan Finger LSB |  |
| 41 | Free Scan Finger HSB |  |
| 42 | Free Scan Finger USB |  |
| 43 | Free Scan Finger MSB |  |
| 44 | RFU |  |
| 45 | RFU |  |
| 46 | RFU |  |
| 47 | RFU |  |
| 48 to 63 | Firmware Version String Char 1 to 16 | 16 Char ASCII string |
| 64 to 79 | Protocol Version String Char 1 to 16 | 16 Char ASCII string |
| 80 | LRC |  |
| 81 | Footer | Fixed BBh |

Set Date Time command (02h) (with payload) (write flag)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total length (LSB) | 13h (19d) |
| 2 | Total length (HSB) | 00h |
| 3 | Command Or Response | 01h (command) |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 02h |
| 7 | Response Required? | 00h (required) |
| 8 | Payload length (LSB) | 07h |
| 9 | Payload length (HSB) | 00h |
| 10 | Hour | 00h to 17h |
| 11 | Minute | 00h to 3Bh |
| 12 | Second | 00h to 3Bh |
| 13 | Day | 01h to 7h |
| 14 | Date | 01h to 1Fh |
| 15 | Month | 01h to 0Ch |
| 16 | Year | 00h to 63h |
| 17 | LRC |  |
| 18 | Footer | Fixed BBh |

## Data handling rules

Data types having more than 8bits (1bytes) like word and long are identified and broken down as per follow in this document.

Multiple byte are organized as little endian convention. If data type of long having value of AABBCCDDh. Then AA is MSB, BB is USB, CC is HSB and DD is LSB. More information on https://en.wikipedia.org/wiki/Endianness.

# Predefined Macros

## FP module Error flags

| Error flags | Number |
| --- | --- |
| FP success | 61h |
| Scan success | 62h |
| Scan fail | 63h |
| Finger not exist | 69h |
| Finger not match | 6Ah |
| Finger not good | 6Bh |
| Command timeout | 6Ch |
| Memory full | 6Dh |
| Finger id already exists | 6Eh |
| Finger Limit | 72h |
| Continue | 74h |
| Command not supported /parameter not matching | 75h |
| Invalid id | 76h |
| Matching timeout | 7Ah |
| Busy | 80h |
| Canceled | 81h |
| Data error | 82h |
| Data OK | 83h |
| Finger already exists | 86h |
| Duress finger | 91h |
| Module locked | A1h |
| Id rejected | 90h |
| Finger limit | 94h |
| Fake finger | B0h |

## FP module Parameter List

| Parameter | Parameter Number | Possible Values |
| --- | --- | --- |
| Finger Placement Time Out | 62h | *3Ah: 10 seconds 3Bh … 44h: 20 seconds |
| Template Size | 64h | 256d to *384d (in multiple of 32) |
| Security Level | 66h | 33h: 1/10000 34h: 1/100000 35h: 1/1000000 36h: 1/10000000 37h: 1/100000000 43h: 3/10000 44h: 3/100000 45h: 3/1000000 46h: 3/10000000 47h: 3/100000000 *50h: Automatic Normal 51h: Automatic Secure 52h: Automatic More Secure |
| Number Of Enrolled Finger | 73h (read only) | Will return number of free scan fingers |
| Available Fingers | 74h (read only) | Will return free space |
| Maximum rotation | 78h | 31h: 15 degree *32h: 30 degree 33h: 45 degree 34h: 60 degree 35h: 75 degree 36h: 90 degree |
| Sensor Sensitivity | 80h | 30h: least sensitive .. *35: sensitivity 5 37h: most sensitive |
| Image quality | 81h | *35h: moderate V2 36h: strong V2 37h: strongest V2 |
| Free Scan | 84h | 0h: Disabled 10d to 60d: Enabled with timeout in seconds FFFFFFFFh : infinite |
| Matching Timeout | 88h | *33h: 3 seconds … 44h: 20 seconds |
| Lightning condition | 90h | *30h: out door 31h: in door |
| Fast mode | 93h | 30h: normal 31h: fast 1 32h: fast 2 33h: fast 3 34h: fast 4 35h: fast 5 *36h: auto |
| Template Type | 96h | 30h: Suprema *31h: ISO |

## Physical Address For Inputs

Available Inputs Per Controllers, other values are RFU / invalid.

BIOSTAMP_3S / BIOSCRIBE_3S :: Reader Port 1, 2. Switch 1. Door Sensor 1. Temper 1 ,2. GPI 1,2.

TWINXS_3S :: Reader Port 1,2,3,4. Switch 1,2. Door Sensor 1,2. Temper 1,2,3,4. GPI 1,2,3,4,5,6,7,8.

| Input | Address |
| --- | --- |
| GPI 1 | 1d |
| GPI 2 | 2d |
| GPI 3 | 3d |
| GPI 4 | 4d |
| GPI 5 | 5d |
| GPI 6 | 6d |
| GPI 7 | 7d |
| GPI 8 | 8d |
| GPI 9 | 9d |
| GPI 10 | 10d |
| GPI 11 | 11d |
| GPI 12 | 12d |
| Reader Port 1 | 17d |
| Reader Port 2 | 18d |
| Reader Port 3 | 19d |
| Reader Port 4 | 20d |
| Reader Port 5 | 21d |
| Reader Port 6 | 22d |
| Reader Port 7 | 23d |
| Reader Port 8 | 24d |
| Temper 1 | 25d |
| Temper 2 | 26d |
| Temper 3 | 27d |
| Temper 4 | 28d |
| Temper 5 | 29d |
| Temper 6 | 30d |
| Temper 7 | 31d |
| Temper 8 | 32d |
| Switch 1 | 33d |
| Switch 2 | 34d |
| Switch 3 | 35d |
| Switch 4 | 36d |
| Switch 5 | 37d |
| Switch 6 | 38d |
| Switch 7 | 39d |
| Switch 8 | 40d |
| Door Sensor 1 | 41d |
| Door Sensor 2 | 42d |
| Door Sensor 3 | 43d |
| Door Sensor 4 | 44d |
| Door Sensor 5 | 45d |
| Door Sensor 6 | 46d |
| Door Sensor 7 | 47d |
| Door Sensor 8 | 48d |

## Physical Address For Outputs

Available Outputs Per controllers. Other values are RFU / invalid

BIOSTAMP_3S / BISCRIBE_3S :: Lock 1. GPO 1.

TWINXS_3S :: Relay 1,2,3,4,5,6,7,8.

| Output | Address |
| --- | --- |
| Relay 1 / Lock 1 | 51d |
| Relay 2 / GPO 1 | 52d |
| Relay 3 | 53d |
| Relay 4 | 54d |
| Relay 5 | 55d |
| Relay 6 | 56d |
| Relay 7 | 57d |
| Relay 8 | 58d |

## Software Event Codes

| Event | Address |
| --- | --- |
| Access Allowed | 1d |
| Exceptional Allow | 2d |
| Soft Local APB | 3d |
| Soft Global APB | 4d |
| Soft Local Timed APB | 5d |
| Soft Global Timed APB | 6d |
| Soft Conditional APB | 7d |
| Soft Concentric APB | 8d |
| Soft Matrix APB | 9d |
| Soft Local Access Limit | 10d |
| Soft Global Access Limit | 11d |
| Soft Local User Absent | 12d |
| Soft Global User Absent | 13d |
| Soft Custodian Not Found | 14d |
| Soft Local Occupancy | 15d |
| Soft Global Occupancy | 16d |
| Soft Local Sentry Check | 17d |
| Soft Global Sentry Check | 18d |
|  | 19d |
|  | 20d |
|  | 21d |
|  | 22d |
|  | 23d |
|  | 24d |
|  | 25d |
|  | 26d |
|  | 27d |
|  | 28d |
|  | 29d |
|  | 30d |
|  | 31d |
|  | 32d |
| Unknown Card format | 33d |
| Invalid Door Config | 34d |
| User Not Found | 35d |
| Invalid Site Code | 36d |
| Validity Not Started | 37d |
| Validity Expired | 38d |
| Not On This Door | 39d |
| Out Of Time Zone | 40d |
| Due To Holiday | 41d |
| Due To Week Off | 42d |
| Wait For Escort | 43d |
| Escort Fail | 44d |
| User Blocked | 45d |
| Finger Scan Failed | 46d |
| Finger not Found | 47d |
| Finger not matched | 48d |
| Finger not good | 49d |
| Finger timeout | 50d |
| Fp command error | 51d |
| Finger before card | 52d |
| Finger not allowed | 53d |
| Invalid pin | 54d |
| Pin entry timeout | 55d |
| Forcefully closed | 56d |
| Switch pressed | 57d |
| Dotl event | 58d |
| ForceFull event | 59d |
| Server deny | 60d |
| Server fail | 61d |
| Local apb fail | 62d |
| Global apb fail | 63d |
| Local timed apb fail | 64d |
| Global timed apb fail | 65d |
| Conditional apb fail | 66d |
| Concentric apb fail | 67d |
| Matrix apb fail | 68d |
| Local access limit | 69d |
| Global access limit | 70d |
| Local user absent | 71d |
| Global user absent | 72d |
| No custodian found | 73d |
| Local occupancy limit | 74d |
| Global occupancy limit | 75d |
| Local sentry check fail | 76d |
| Global sentry check fail | 77d |
| Further authentication needed | 78d |
| Door interlocked | 79d |
| Sentry can not pass | 80d |
| Fake finger | 81d |
| Daily Item Limit Exceeded | 101d |
| Weekly Item Limit Exceeded | 102d |
| Monthly Item Limit Exceeded | 103d |
| TimeZone wise Item Limit Exceeded | 104d |
| Balance Limit Exceeded | 105d |

## Response Code List

| Output | Address |
| --- | --- |
| Success /OK | 1d |
| Invalid Frame Length | 2d |
| LRC Check Fail | 3d |
| Invalid Data Arguments | 4d |
| Invalid Payload Length | 5d |
| Invalid Response | 6d |
| Invalid Header | 7d |
| Invalid Footer | 8d |
| Too Large Frame | 9d |
| Long Timeout | 10d |
| Unknown Command | 11d |
| No CID Set | 12d |
| Write Error | 13d |
| Read Error | 14d |
| User Not Found | 15d |
| Unknown Error | 16d |
| Invalid Range | 17d |
| Require Read only Operation | 18d |
| Require Write Only Operation | 19d |
| Invalid Sequence | 20d |
| CRC Match Fail | 21d |
| Device Busy | 22d |
| Command Timeout | 23d |
| Data Full | 24d |
| Not Ready | 25d |

# Function Code (With Payloads)

## Get Status (01h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h / 02h As Identifier string device will send this frame as command. |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | 01h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | IPv4 address segment 1 | C0h (if ip is 192.100.100.90) |
| 17 | IPv4 address segment 2 | 64h |
| 18 | IPv4 address segment 3 | 64h |
| 19 | IPv4 address segment 4 | 5Ah |
| 20 | CID LSB | YYh (if CID is XXYYh) |
| 21 | CID HSB | XXh |
| 22 | Year | 00h to 63h |
| 23 | Month | 01h to 0Ch |
| 24 | Date | 01h to 1Fh |
| 25 | Day | 01h to 7h |
| 26 | Hour | 00h to 17h |
| 27 | Minute | 00h to 3Bh |
| 28 | Second | 00h to 3Bh |
| 29 | Communication Interface | 1h: ETH 2h: WIFI 3h: GSM |
| 30 | RFU | 0h |
| 31 | RFU | 0h |
| 32 | Total Users (LSB) | DDh (four byte value AABBCCDDh) |
| 33 | Total Users (HSB) | CCh |
| 34 | Total Users (USB) | BBh |
| 35 | Total Users (MSB) | AAh |
| 36 | Total Trans. (LSB) | DDh (four byte value AABBCCDDh) |
| 37 | Total Trans. (HSB) | CCh |
| 38 | Total Trans. (USB) | BBh |
| 39 | Total Trans. (MSB) | AAh |
| 40 | Free Scan Finger LSB |  |
| 41 | Free Scan Finger HSB |  |
| 42 | Free Scan Finger USB |  |
| 43 | Free Scan Finger MSB |  |
| 44 | RFU |  |
| 45 | RFU |  |
| 46 | RFU |  |
| 47 | RFU |  |
| 48 to 63 | Firmware Version String Char 1 to 16 | 16 Char ASCII string |
| 64 to 79 | Protocol Version String Char 1 to 16 | 16 Char ASCII string |
| 80 | LRC |  |
| 81 | Footer | Fixed BBh |

## Set Date & Time (02h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total length (LSB) | YYh |
| 2 | Total length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 02h |
| 7 | Response Required? /Response | 01h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Hour | 00h to 17h |
| 11 | Minute | 00h to 3Bh |
| 12 | Second | 00h to 3Bh |
| 13 | Day | 01h to 7h |
| 14 | Date | 01h to 1Fh |
| 15 | Month | 01h to 0Ch |
| 16 | Year | 00h to 63h |
| 17 | LRC |  |
| 18 | Footer | Fixed BBh |

## Add User (Time Zone Based ACCESS) (03h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 03h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh |
| 9 | Payload Length(HSB) | XXh |
| 10 | User Checking Bypass | 0h: No Search 1h: Search |
| 11 | UID LSB | 78h(If UID is 0x12345678) |
| 12 | UID HSB | 56h |
| 13 | UID USB | 34h |
| 14 | UID MSB | 12h |
| 15 | EXTD ID LSB (for 64 bit CardID only) | 78h(If UID is 0x12345678) |
| 16 | EXTD ID HSB | 56h |
| 17 | EXTD ID USB | 34h |
| 18 | EXTD ID MSB | 12h |
| 19 | Time Zone Wise / Exclusive | 1h: Tz wise (fixed) |
| 20 | User Type | 0d to 32d (0h to 20h) |
| 21 | Week Off group number LSB | Week Off Group ID |
| 22 | Week Off group number HSB |  |
| 23 | Holiday Access group LSB | Holiday Group ID |
| 24 | Holiday Access group HSB |  |
| 25 | Start Year | Validity start time |
| 26 | Start Month |  |
| 27 | Start Date |  |
| 28 | Start Hour |  |
| 29 | Start Minute |  |
| 30 | End Year | Expiry time |
| 31 | End Month |  |
| 32 | End Date |  |
| 33 | End Hour |  |
| 34 | End Minute |  |
| 35 | Pin LSB | 4Eh (if pin is 12345678d == BC614Eh) |
| 36 | Pin HSB | 61h |
| 37 | Pin USB | BCh |
| 38 | Pin MSB | 00h |
| 39 to 70 | User Name (32 Bytes) | “Name of User” ASCII string null terminated. |
| 71 to 86 | Employee ID (16 Bytes) | ASCII string null terminated. |
| 87 | Time Zone Group ID 1 LSB | Group No of Time Zone. If any time zones not used, Fill 0h. Max 8 time zones. Canteen Management System : Byte 87 : Item Group Number (1d to 100d) BYTE 102 : Prepaid (1) / Postpaid (0) |
| 88 | Time Zone Group ID 1 HSB |  |
| 89 | Time Zone Group ID 2 LSB |  |
| 90 | Time Zone Group ID 2 HSB |  |
| 91 | Time Zone Group ID 3 LSB |  |
| 92 | Time Zone Group ID 3 HSB |  |
| 93 | Time Zone Group ID 4 LSB |  |
| 94 | Time Zone Group ID 4 HSB |  |
| 95 | Time Zone Group ID 5 LSB |  |
| 96 | Time Zone Group ID 5 HSB |  |
| 97 | Time Zone Group ID 6 LSB |  |
| 98 | Time Zone Group ID 6 HSB |  |
| 99 | Time Zone Group ID 7 LSB |  |
| 100 | Time Zone Group ID 7 HSB |  |
| 101 | Time Zone Group ID 8 LSB |  |
| 102 | Time Zone Group ID 8 HSB |  |
| 103 | Escort User ID LSB |  |
| 104 | Escort User ID HSB |  |
| 105 | Escort User ID USB |  |
| 106 | Escort User ID MSB |  |
| 107 | Escort User EXTD ID LSB (for 64 bit CardID only) |  |
| 108 | Escort User EXTD ID HSB |  |
| 109 | Escort User EXTD ID USB |  |
| 110 | Escort User EXTD ID MSB |  |
| 111 | LRC | LRC |
| 112 | Footer | Fixed BBh |

## Add user (Exclusive Gate wise Access) (04h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 04h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh |
| 9 | Payload Length(HSB) | XXh |
| 10 | User Checking Bypass | 0h: No Search 1h: Search |
| 11 | UID LSB | 78h(If UID is 0x12345678) |
| 12 | UID HSB | 56h |
| 13 | UID USB | 34h |
| 14 | UID MSB | 12h |
| 15 | EXTD ID LSB (for 64 bit CardID only) | 78h(If UID is 0x12345678) |
| 16 | EXTD ID HSB | 56h |
| 17 | EXTD ID USB | 34h |
| 18 | EXTD ID MSB | 12h |
| 19 | Time Zone Wise / Exclusive | 2h: Exclusive Gate wise (fixed) |
| 20 | User Type | 0d to 32d (1h to 20h) |
| 21 | Week Off group number LSB | Week Off Group ID |
| 22 | Week Off group number HSB |  |
| 23 | Holiday Access group LSB | Holiday Group ID |
| 24 | Holiday Access group HSB |  |
| 25 | Start Year | Validity start time |
| 26 | Start Month |  |
| 27 | Start Date |  |
| 28 | Start Hour |  |
| 29 | Start Minute |  |
| 30 | End Year | Expiry time |
| 31 | End Month |  |
| 32 | End Date |  |
| 33 | End Hour |  |
| 34 | End Minute |  |
| 35 | Pin LSB | 4Eh (if pin is 12345678d == BC614Eh) |
| 36 | Pin HSB | 61h |
| 37 | Pin USB | BCh |
| 38 | Pin MSB | 00h |
| 39 to 70 | User Name (32 Bytes) | “Name of User” ASCII string null terminated. |
| 71 to 86 | Employee ID (16 Bytes) | ASCII string null terminated. |
| 87 | Door 1 Start Time Hour | 0d to 23d |
| 88 | Door 1 Start Time Minute | 0d to 59d |
| 89 | Door 1 End Time Hour | 0d to 23d |
| 90 | Door 1 End Time Minute | 0d to 59d |
| 91 | Door 2 Start Time Hour |  |
| 92 | Door 2 Start Time Minute |  |
| 93 | Door 2 End Time Hour |  |
| 94 | Door 2 End Time Minute |  |
| 95 | Door 3 Start Time Hour |  |
| 96 | Door 3 Start Time Minute |  |
| 97 | Door 3 End Time Hour |  |
| 98 | Door 3 End Time Minute |  |
| 99 | Door 4 Start Time Hour |  |
| 100 | Door 4 Start Time Minute |  |
| 101 | Door 4 End Time Hour |  |
| 102 | Door 4 End Time Minute |  |
| 103 | Escort User UID LSB |  |
| 104 | Escort User UID HSB |  |
| 105 | Escort User UID USB |  |
| 106 | Escort User UID MSB |  |
| 107 | Escort User EXTD ID LSB (for 64 bit CardID only) |  |
| 108 | Escort User EXTD ID HSB |  |
| 109 | Escort User EXTD ID USB |  |
| 110 | Escort User EXTD ID MSB |  |
| 111 | LRC | LRC |
| 112 | Footer | Fixed BBh |

## Upload File Header (05h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 05h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh |
| 9 | Payload Length(HSB) | XXh |
| 10 | Mode | 0h: edit file 1h: create file 2h: delete file |
| 11 | RFU |  |
| 12 | RFU |  |
| 13 | RFU |  |
| 14 | RFU |  |
| 15 | File Size in Bytes LSB | E8h (If file size is 1000d = 3E8h) |
| 16 | File Size in Bytes HSB | 03h |
| 17 | File Size in Bytes USB | 00h |
| 18 | File Size in Bytes MSB | 00h |
| 19 to N - 2 | File name including folder path | Null terminated Ascii string like “USER/aabbccdd.bmp\0” “AUDIO/allow.wav\0” “AUDIO/deny.wav\0” “complogo.bmp\0” “SS/SS01.jpg\0” |
| N - 1 | LRC | LRC |
| N | Footer | Fixed BBh |

## Upload File Data (06h)

File must be created using 05h command first.

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 06h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh (max 1024 for this command) |
| 9 | Payload Length(HSB) | XXh |
| 10 | Sequence Number LSB | DDh (if seq. No. = AABBCCDDh) |
| 11 | Sequence Number HSB | CCh |
| 12 | Sequence Number USB | BBh |
| 13 | Sequence Number MSB | AAh |
| 15 to N - 2 | File Data | Binary Data max 1024 bytes |
| N - 1 | LRC | LRC |
| N | Footer | Fixed BBh |

## Delete Particular User (07h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 07h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh |
| 9 | Payload Length(HSB) | XXh |
| 10 | UID LSB | DDh (if card id is AABBCCDDh) |
| 11 | UID HSB | CCh |
| 12 | UID USB | BBh |
| 13 | UID MSB | AAh |
| 14 | EXTD ID LSB (for 64 bit CardID only) | DDh (if card id is AABBCCDDh) |
| 15 | EXTD ID HSB | CCh |
| 16 | EXTD ID USB | BBh |
| 17 | EXTD ID MSB | AAh |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

## Delete All Users (I.E. DB delete all) (08h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 08h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | 00h |
| 9 | Payload Length(HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Add User Exception (09h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 09h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Search or No Search | 0h: do not search add new 1h: search for existing user |
| 11 | UID LSB |  |
| 12 | UID HSB |  |
| 13 | UID USB |  |
| 14 | UID MSB |  |
| 15 | Site Code LSB |  |
| 16 | Site Code HSB |  |
| 17 | Site Code USB |  |
| 18 | Site Code MSB |  |
| 19 | Door 1 Block Access / Forcefully Allow | 0d: nothing 1d: block 2d: Force Allow |
| 20 | Door 1 Exception Start Year | 0d to 99d |
| 21 | Door 1 Exception Start Month | 1d t o12d |
| 22 | Door 1 Exception Start Date | 1d to 31d |
| 23 | Door 1 Exception Start Hour | 0d to 23d |
| 24 | Door 1 Exception Start Minute | 0d to 59d |
| 25 | Door 1 Exception End Year | 0d to 99d |
| 26 | Door 1 Exception End Month | 1d to 12d |
| 27 | Door 1 Exception End Date | 1d to 31d |
| 28 | Door 1 Exception End Hour | 0d to 23d |
| 29 | Door 1 Exception End Minute | 0d to 59d |
| 30 | Door 2 Block Access / Forcefully Allow | 0d: nothing 1d: block 2d: Force Allow |
| 31 | Door 2 Exception Start Year | 0d to 99d |
| 32 | Door 2 Exception Start Month | 1d t o12d |
| 33 | Door 2 Exception Start Date | 1d to 31d |
| 34 | Door 2 Exception Start Hour | 0d to 23d |
| 35 | Door 2 Exception Start Minute | 0d to 59d |
| 36 | Door 2 Exception End Year | 0d to 99d |
| 37 | Door 2 Exception End Month | 1d to 12d |
| 38 | Door 2 Exception End Date | 1d to 31d |
| 39 | Door 2 Exception End Hour | 0d to 23d |
| 40 | Door 2 Exception End Minute | 0d to 59d |
| 41 | Door 3 Block Access / Forcefully Allow | 0d: nothing 1d: block 2d: Force Allow |
| 42 | Door 3 Exception Start Year | 0d to 99d |
| 43 | Door 3 Exception Start Month | 1d t o12d |
| 44 | Door 3 Exception Start Date | 1d to 31d |
| 45 | Door 3 Exception Start Hour | 0d to 23d |
| 46 | Door 3 Exception Start Minute | 0d to 59d |
| 47 | Door 3 Exception End Year | 0d to 99d |
| 48 | Door 3 Exception End Month | 1d to 12d |
| 49 | Door 3 Exception End Date | 1d to 31d |
| 50 | Door 3 Exception End Hour | 0d to 23d |
| 51 | Door 3 Exception End Minute | 0d to 59d |
| 52 | Door 4 Block Access / Forcefully Allow | 0d: nothing 1d: block 2d: Force Allow |
| 53 | Door 4 Exception Start Year | 0d to 99d |
| 54 | Door 4 Exception Start Month | 1d t o12d |
| 55 | Door 4 Exception Start Date | 1d to 31d |
| 56 | Door 4 Exception Start Hour | 0d to 23d |
| 57 | Door 4 Exception Start Minute | 0d to 59d |
| 58 | Door 4 Exception End Year | 0d to 99d |
| 59 | Door 4 Exception End Month | 1d to 12d |
| 60 | Door 4 Exception End Date | 1d to 31d |
| 61 | Door 4 Exception End Hour | 0d to 23d |
|  | Door 4 Exception End Minute | 0d to 59d |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Delete User Exception (0Ah)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 0Ah |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 08h |
| 9 | Payload length (HSB) | 00h |
| 10 | UID LSB |  |
| 11 | UID HSB |  |
| 12 | UID USB |  |
| 13 | UID MSB |  |
| 14 | Site Code LSB |  |
| 15 | Site Code HSB |  |
| 16 | Site Code USB |  |
| 17 | Site Code MSB |  |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

## Scan Live Finger (0Bh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | 0Bh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | RFU | 0h |
| 11 | Returned Template Size HSB | 01h (if 384d = 180h) |
| 12 | Returned Template Size LSB | 80h |
| 13 | Scanned Quality Score | Max 100d (in percentage) |
| 14 to N-2 | First Byte of Template Data |  |
| … | … |  |
| N - 2 | Last Byte of Template Data | Max 512d bytes |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Set finger Template (0Ch)

Finger Template Length may be variable. And depends upon finger size setting in fppara.

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 0Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | No Of Templates in this command | 1h or 2h |
| 11 | UID LSB |  |
| 12 | UID HSB |  |
| 13 | UID USB |  |
| 14 | UID MSB |  |
| 15 | EXTD ID LSB (for 64 bit CardID only) |  |
| 16 | EXTD ID HSB |  |
| 17 | EXTD ID USB |  |
| 18 | EXTD ID MSB |  |
| 19 | Template 1 Index | 0h to 9h |
| 20 | Template 1 Scope / Template 2 index in Read command | 0h: local 1h: free scan / 0h to 9h |
| 21 | Template 1 size LSB | 80h (if 384d = 180h) |
| 22 | Template 1 size HSB | 01h |
| 23.. to M | Template 1 Data byte 0 |  |
|  | … |  |
| M | Template 1 Data byte last |  |
| M+1 | Template 2 Index |  |
| M+2 | Template 2 Scope |  |
| M+3 | Template 2 size LSB |  |
| M+4 | Template 2 size HSB |  |
| M+5 to N - 2 | Template 2 Data byte 0 |  |
| … | … |  |
| N - 2 | Template 2 Data byte last |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Delete Finger Template (0Dh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 0Dh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Scope | 0h: local 1h: free scan |
| 11 | Template Index (RFU) | 0h: fixed |
| 12 | UID LSB |  |
| 13 | UID HSB |  |
| 14 | UID USB |  |
| 15 | UID MSB |  |
| 16 | EXTD ID LSB (for 64 bit CardID only) |  |
| 17 | EXTD ID HSB |  |
| 18 | EXTD ID USB |  |
| 19 | EXTD ID MSB |  |
| 20 | LRC |  |
| 21 | Footer | Fixed BBh |

## Delete All Finger (0Eh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 0Eh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 01h |
| 9 | Payload length (HSB) | 00h |
| 10 | Scope | 0h: local 1h: free scan + local |
| 11 | LRC |  |
| 12 | Footer | Fixed BBh |

## Read / Write FP module Parameters (0Fh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h / 01h |
| 6 | FC | 0Fh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Parameter No. | See FP para list |
| 11 | Value LSB |  |
| 12 | Value HSB |  |
| 13 | Value USB |  |
| 14 | Value MSB |  |
| 15 | LRC |  |
| 16 | Footer | Fixed BBh |

## Sync Time Zones (10h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 10h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | TZ 1 Start Hour (fixed read only) | 0d |
| 11 | TZ 1 Start Minute (fixed read only) | 0d |
| 12 | TZ 1 End Hour (fixed read only) | 23d |
| 13 | TZ 1 End Minute (fixed read only) | 60d (60d value only valid for this byte) |
| 14 to … | … | 0d to 59d |
|  | … | 0d to 59d |
|  | … | 0d to 23d |
|  | … | 0d to 59d |
| N - 5 | TZ 127 Start Hour | 0d to 59d |
| N - 4 | TZ 127 Start Minute | 0d to 59d |
| N - 3 | TZ 127 End Hour | 0d to 23d |
| N - 2 | TZ 127 End Minute | 0d to 59d |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Add Time Zone Access Group (11h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 11h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
|  | No Of Time Zone Groups In This Frame | 1d to 50d |
| 10 | Time Zone Access Group Number LSB | YYh (XXYYh = 2d to 1024d) (1d is read only) |
| 11 | Time Zone Access Group Number HSB | XXh |
| 12 | 1st Time Zone for Gate 1 | 1d to 127d |
| 13 | 2nd Time Zone for Gate 1 | 1d to 127d |
| 14 | 3rd Time Zone for Gate 1 | 1d to 127d |
| 15 | 4th Time Zone for Gate 1 | 1d to 127d |
| 16 | Allow / Deny for Gate 1 | 00h : allow 01h : deny |
| 17 | 1st Time Zone for Gate 2 |  |
| 18 | 2nd Time Zone for Gate 2 |  |
| 19 | 3rd Time Zone for Gate 2 |  |
| 20 | 4th Time Zone for Gate 2 |  |
| 21 | Allow / Deny for Gate 2 |  |
| 22 | 1st Time Zone for Gate 3 |  |
| 23 | 2nd Time Zone for Gate 3 |  |
| 24 | 3rd Time Zone for Gate 3 |  |
| 25 | 4th Time Zone for Gate 3 |  |
| 26 | Allow / Deny for Gate 3 |  |
| 27 | 1st Time Zone for Gate 4 |  |
| 28 | 2nd Time Zone for Gate 4 |  |
| 29 | 3rd Time Zone for Gate 4 |  |
| 30 | 4th Time Zone for Gate 4 |  |
| 31 | Allow / Deny for Gate 4 |  |
| 32 | LRC |  |
| 33 | Footer | Fixed BBh |

## Delete All Time Zone Groups (12h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 12h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Sync Week Off Groups (13h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 13h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Group 1 Week 1 Days | 0b0XXXXXXXX Where each X represents days 0b0SFTWTMS 1b: day on 0b: day off |
| 11 | Group 1 Week 2 Days |  |
| 12 | Group 1 Week 3 Days |  |
| 13 | Group 1 Week 4 Days |  |
| 14 | Group 1 Week 5 Days |  |
| 15 | … |  |
| 16 | … |  |
| 17 | … |  |
| 18 | … |  |
| 19 to 164 | … |  |
| 165 | Group 32 Week 1 Days |  |
| 166 | Group 32 Week 2 Days |  |
| 167 | Group 32 Week 3 Days |  |
| 168 | Group 32 Week 4 Days |  |
| 169 | Group 32 Week 5 Days |  |
| 170 | LRC |  |
| 171 | Footer | Fixed BBh |

## Set Holiday Group (14h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 14h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Holiday Group No. | 1h to 32h |
| 11 | Date 1 | 1d to 31d |
| 12 | Month 1 | 1d to 12d |
| 13 | Year 1 | 00d to 99d |
| 14 | Date 2 |  |
| 15 | Month 2 |  |
| 16 | Year 2 |  |
| 17 to ... | … |  |
| N - 5 | … |  |
| N - 4 | Date 128 |  |
| N - 3 | Month 128 |  |
| N - 2 | Year 128 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Delete All Holiday Groups (15h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 15h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Controller Setup (16h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 16h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | CID LSB | E8h (If CID is 1000d (3E8h)) |
| 11 | CID HSB | 03h |
| 12 | Device Mode | 0h: Controller mode 1h: Wiegand Reader Mode |
| 13 | Reader Mode | 0h: Default Mode 1h: In Mode 2h: Out Mode 3h: Toggle Mode 4h: Null Mode |
| 14 | RFU | 0h |
| 15 | RFU |  |
| 16 | RFU | 0h |
| 17 | RFU |  |
| 18 | RFU | 0h |
| 19 | RFU | 0h |
| 20 | RFU | 0h |
| 21 | Transaction Frame Type | 00h: Old Type Transaction Frame 01h: New Type Transaction Frame |
| 22 | LRC |  |
| 23 | Footer | Fixed BBh |

## Reader Setup (17h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 17h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | CID LSB | E8h (If CID is 1000d (3E8h)) |
| 11 | CID HSB | 03h |
| 12 | Device Mode | 1h: Wiegand Reader Mode |
| 13 | RFU | 0h |
| 14 | Wigand Pulse Period LSB | 50d to 1000d (micro seconds) |
| 15 | Wigand Pulse Period HSB |  |
| 16 | Wigand Pulse Interval LSB | 1000d to 10000d (micro seconds) |
| 17 | Wigand Pulse Interval HSB |  |
| 18 | Authentication Type in Wiegand Reader Mode | 01h: Card Only 02h: Card + Pin 03h: Card / Pin 04h: Card + Finger 05h: Card + Pin + Finger 06h: Only Finger 07h: Card / Finger |
| 19 | Transaction Push Enable? | 0h: disable 1h: enable |
| 20 | RFU | 0h |
| 21 | Transaction Frame Type | 00h: Old Type Transaction Frame 01h: New Type Transaction Frame |
| 22 | LRC |  |
| 23 | Footer | Fixed BBh |

## Door Setup (18h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 18h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door 1 Enable / Disable? | 0h : disable 1h : enable |
| 11 | Door 1 Entry Reader Number | 1h to 8h |
| 12 | Door 1 Exit Reader Number |  |
| 13 | Door 1 Entry Mode | 01h: Card Only 02h: Card + Pin 03h: Card / Pin 04h: Card + Finger 05h: Card + Pin + Finger 06h: Only Finger 07h: Card / Finger |
| 14 | Door 1 Exit Mode |  |
| 15 | Door 1 Entry Relay | See Output List |
| 16 | Door 1 Exit Relay |  |
| 17 | Door 1 Exit Verification for TZ | 0h: disable *1h: enable |
| 18 | Door 1 Entry Open Time LSB | YYh (5d to 9999d) (seconds) |
| 19 | Door 1 Entry Open Time HSB | XXh |
| 20 | Door 1 Exit Open Time LSB |  |
| 21 | Door 1 Exit Open Time HSB |  |
| 22 | Door 1 Sensor Time Limit LSB | YYh (5d to 9999d) (seconds) |
| 23 | Door 1 Sensor Time Limit HSB | XXh (0d to disable feature) |
| 24 | Door 1 Monitor Enable/Disable? | 0h : disable 1h : enable |
| 25 | RFU | 0h |
| 26 | Door 2 Enable / Disable? | 0h : disable 1h : enable |
| 27 | Door 2 Entry Reader Number |  |
| 28 | Door 2 Exit Reader Number |  |
| 29 | Door 2 Entry Mode |  |
| 30 | Door 2 Exit Mode |  |
| 31 | Door 2 Entry Relay |  |
| 32 | Door 2 Exit Relay |  |
| 33 | Door 2 Exit Verification for TZ |  |
| 34 | Door 2 Entry Open Time LSB |  |
| 35 | Door 2 Entry Open Time HSB |  |
| 36 | Door 2 Exit Open Time LSB |  |
| 37 | Door 2 Exit Open Time HSB |  |
| 38 | Door 2 Sensor Time Limit LSB |  |
| 39 | Door 2 Sensor Time Limit HSB |  |
| 40 | Door 2 Monitor Enable/Disable? | 0h : disable 1h : enable |
| 41 | RFU | 0h |
| 42 | Door 3 Enable / Disable? |  |
| 43 | Door 3 Entry Reader Number |  |
| 44 | Door 3 Exit Reader Number |  |
| 45 | Door 3 Entry Mode |  |
| 46 | Door 3 Exit Mode |  |
| 47 | Door 3 Entry Relay |  |
| 48 | Door 3 Exit Relay |  |
| 49 | Door 3 Exit Verification for TZ |  |
| 50 | Door 3 Entry Open Time LSB |  |
| 51 | Door 3 Entry Open Time HSB |  |
| 52 | Door 3 Exit Open Time LSB |  |
| 53 | Door 3 Exit Open Time HSB |  |
| 54 | Door 3 Sensor Time Limit LSB |  |
| 55 | Door 3 Sensor Time Limit HSB |  |
| 56 | Door 3 Monitor Enable/Disable? | 0h : disable 1h : enable |
| 57 | RFU | 0h |
| 58 | Door 4 Enable / Disable? |  |
| 59 | Door 4 Entry Reader Number |  |
| 60 | Door 4 Exit Reader Number |  |
| 61 | Door 4 Entry Mode |  |
| 62 | Door 4 Exit Mode |  |
| 63 | Door 4 Entry Relay |  |
| 64 | Door 4 Exit Relay |  |
| 65 | Door 4 Exit Verification for TZ |  |
| 66 | Door 4 Entry Open Time LSB |  |
| 67 | Door 4 Entry Open Time HSB |  |
| 68 | Door 4 Exit Open Time LSB |  |
| 69 | Door 4 Exit Open Time HSB |  |
| 70 | Door 4 Sensor Time Limit LSB |  |
| 71 | Door 4 Sensor Time Limit HSB |  |
| 72 | Door 4 Monitor Enable/Disable? | 0h : disable 1h : enable |
| 73 |  |  |
| 74 | LRC |  |
| 75 | Footer | Fixed BBh |

## Door Mode Exceptions (19h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 19h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door 1 Enabled Days | 0b0XXXXXXX (0b0SMTWTFS) |
| 11 | Door 1 Mode Sunday | 00h: Default Gate wise 01h: Card Only 02h: Card + Pin 03h: Card / Pin 04h: Card + Finger 05h: Card + Pin + Finger 06h: Only Finger 07h: Card / Finger |
| 12 | Door 1 Mode Monday |  |
| 13 | Door 1 Mode Tuesday |  |
| 14 | Door 1 Mode Wednesday |  |
| 15 | Door 1 Mode Thursday |  |
| 16 | Door 1 Mode Friday |  |
| 17 | Door 1 Mode Saturday |  |
| 18 | Door 1 Sunday Start 1 Hour |  |
| 19 | Door 1 Sunday Start 1 Minute |  |
| 20 | Door 1 Sunday End 1 Hour |  |
| 21 | Door 1 Sunday End 1 Minute |  |
| 22 | Door 1 Sunday Start 2 Hour |  |
| 23 | Door 1 Sunday Start 2 Minute |  |
| 24 | Door 1 Sunday End 2 Hour |  |
| 25 | Door 1 Sunday End 2 Minute |  |
| 26 | Door 1 Monday Start 1 Hour |  |
| 27 | Door 1 Monday Start 1 Minute |  |
| 28 | Door 1 Monday End 1 Hour |  |
| 29 | Door 1 Monday End 1 Minute |  |
| 30 | Door 1 Monday Start 2 Hour |  |
| 31 | Door 1 Monday Start 2 Minute |  |
| 32 | Door 1 Monday End 2 Hour |  |
| 33 | Door 1 Monday End 2 Minute |  |
| 34 | Door 1 Tuesday Start 1 Hour |  |
| 35 | Door 1 Tuesday Start 1 Minute |  |
| 36 | Door 1 Tuesday End 1 Hour |  |
| 37 | Door 1 Tuesday End 1 Minute |  |
| 38 | Door 1 Tuesday Start 2 Hour |  |
| 39 | Door 1 Tuesday Start 2 Minute |  |
| 40 | Door 1 Tuesday End 2 Hour |  |
| 41 | Door 1 Tuesday End 2 Minute |  |
| 42 | Door 1 Wednesday Start 1 Hour |  |
| 43 | Door 1 Wednesday Start 1 Minute |  |
| 44 | Door 1 Wednesday End 1 Hour |  |
| 45 | Door 1 Wednesday End 1 Minute |  |
| 46 | Door 1 Wednesday Start 2 Hour |  |
| 47 | Door 1 Wednesday Start 2 Minute |  |
| 48 | Door 1 Wednesday End 2 Hour |  |
| 49 | Door 1 Wednesday End 2 Minute |  |
| 50 | Door 1 Thursday Start 1 Hour |  |
| 51 | Door 1 Thursday Start 1 Minute |  |
| 52 | Door 1 Thursday End 1 Hour |  |
| 53 | Door 1 Thursday End 1 Minute |  |
| 54 | Door 1 Thursday Start 2 Hour |  |
| 55 | Door 1 Thursday Start 2 Minute |  |
| 56 | Door 1 Thursday End 2 Hour |  |
| 57 | Door 1 Thursday End 2 Minute |  |
| 58 | Door 1 Friday Start 1 Hour |  |
| 59 | Door 1 Friday Start 1 Minute |  |
| 60 | Door 1 Friday End 1 Hour |  |
| 61 | Door 1 Friday End 1 Minute |  |
| 62 | Door 1 Friday Start 2 Hour |  |
| 63 | Door 1 Friday Start 2 Minute |  |
| 64 | Door 1 Friday End 2 Hour |  |
| 65 | Door 1 Friday End 2 Minute |  |
| 66 | Door 1 Saturday Start 1 Hour |  |
| 67 | Door 1 Saturday Start 1 Minute |  |
| 68 | Door 1 Saturday End 1 Hour |  |
| 69 | Door 1 Saturday End 1 Minute |  |
| 70 | Door 1 Saturday Start 2 Hour |  |
| 71 | Door 1 Saturday Start 2 Minute |  |
| 72 | Door 1 Saturday End 2 Hour |  |
| 73 | Door 1 Saturday End 2 Minute |  |
| 74 to --- | Same for Door 2 |  |
| --- to --- | Same for Door 3 |  |
| --- to N - 2 | Same for Door 4 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Door State Exceptions (1Ah)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 1Ah |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door 1 Enabled Days | 0b0XXXXXXX (0b0SMTWTFS) |
| 11 | Door 1 Mode Sunday | 0h: armed 1h: open 2h: closed |
| 12 | Door 1 Mode Monday |  |
| 13 | Door 1 Mode Tuesday |  |
| 14 | Door 1 Mode Wednesday |  |
| 15 | Door 1 Mode Thursday |  |
| 16 | Door 1 Mode Friday |  |
| 17 | Door 1 Mode Saturday |  |
| 18 | Door 1 Sunday Start 1 Hour |  |
| 19 | Door 1 Sunday Start 1 Minute |  |
| 20 | Door 1 Sunday End 1 Hour |  |
| 21 | Door 1 Sunday End 1 Minute |  |
| 22 | Door 1 Sunday Start 2 Hour |  |
| 23 | Door 1 Sunday Start 2 Minute |  |
| 24 | Door 1 Sunday End 2 Hour |  |
| 25 | Door 1 Sunday End 2 Minute |  |
| 26 | Door 1 Monday Start 1 Hour |  |
| 27 | Door 1 Monday Start 1 Minute |  |
| 28 | Door 1 Monday End 1 Hour |  |
| 29 | Door 1 Monday End 1 Minute |  |
| 30 | Door 1 Monday Start 2 Hour |  |
| 31 | Door 1 Monday Start 2 Minute |  |
| 32 | Door 1 Monday End 2 Hour |  |
| 33 | Door 1 Monday End 2 Minute |  |
| 34 | Door 1 Tuesday Start 1 Hour |  |
| 35 | Door 1 Tuesday Start 1 Minute |  |
| 36 | Door 1 Tuesday End 1 Hour |  |
| 37 | Door 1 Tuesday End 1 Minute |  |
| 38 | Door 1 Tuesday Start 2 Hour |  |
| 39 | Door 1 Tuesday Start 2 Minute |  |
| 40 | Door 1 Tuesday End 2 Hour |  |
| 41 | Door 1 Tuesday End 2 Minute |  |
| 42 | Door 1 Wednesday Start 1 Hour |  |
| 43 | Door 1 Wednesday Start 1 Minute |  |
| 44 | Door 1 Wednesday End 1 Hour |  |
| 45 | Door 1 Wednesday End 1 Minute |  |
| 46 | Door 1 Wednesday Start 2 Hour |  |
| 47 | Door 1 Wednesday Start 2 Minute |  |
| 48 | Door 1 Wednesday End 2 Hour |  |
| 49 | Door 1 Wednesday End 2 Minute |  |
| 50 | Door 1 Thursday Start 1 Hour |  |
| 51 | Door 1 Thursday Start 1 Minute |  |
| 52 | Door 1 Thursday End 1 Hour |  |
| 53 | Door 1 Thursday End 1 Minute |  |
| 54 | Door 1 Thursday Start 2 Hour |  |
| 55 | Door 1 Thursday Start 2 Minute |  |
| 56 | Door 1 Thursday End 2 Hour |  |
| 57 | Door 1 Thursday End 2 Minute |  |
| 58 | Door 1 Friday Start 1 Hour |  |
| 59 | Door 1 Friday Start 1 Minute |  |
| 60 | Door 1 Friday End 1 Hour |  |
| 61 | Door 1 Friday End 1 Minute |  |
| 62 | Door 1 Friday Start 2 Hour |  |
| 63 | Door 1 Friday Start 2 Minute |  |
| 64 | Door 1 Friday End 2 Hour |  |
| 65 | Door 1 Friday End 2 Minute |  |
| 66 | Door 1 Saturday Start 1 Hour |  |
| 67 | Door 1 Saturday Start 1 Minute |  |
| 68 | Door 1 Saturday End 1 Hour |  |
| 69 | Door 1 Saturday End 1 Minute |  |
| 70 | Door 1 Saturday Start 2 Hour |  |
| 71 | Door 1 Saturday Start 2 Minute |  |
| 72 | Door 1 Saturday End 2 Hour |  |
| 73 | Door 1 Saturday End 2 Minute |  |
| 74 to --- | Same for Door 2 |  |
| --- to --- | Same for Door 3 |  |
| --- to N - 2 | Same for Door 4 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Area Setup (1Bh)

RFU command. No need to implement

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Command Header | Fixed AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W | 01h |
| 6 | FC | 1Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload Length (LSB) | YYh |
| 9 | Payload Length(HSB) | XXh |
| 10 | Door 1 Area ID | 1d to 255d |
| 10 | Door 2 Area ID | 1d to 255d |
| 11 | Door 3 Area ID | 1d to 255d |
| 12 | Door 4 Area ID | 1d to 255d |
| 59 | LRC |  |
| 60 | Footer | Fixed BBh |

## Input Definitions (1Ch)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 1Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Input 1 Mapping Address | See List here |
| 11 | Input 1 Mode | 1h: normally opened 2h: normally closed (fixed mode for door sensor) |
| 12 | RFU | 0h |
| 13 | Input 1 Pulse width LSB | YYh (1d to 65000d) |
| 14 | Input 1 Pulse width HSB | XXh (In milliseconds) |
| 15 | Input 2 Mapping Address |  |
| 16 | Input 2 Mode |  |
| 17 | RFU |  |
| 18 | Input 2 Pulse width LSB |  |
| 19 | Input 2 Pulse width HSB |  |
| 20 | Input 3 Mapping Address |  |
| 21 | Input 3 Mode |  |
| 22 | RFU |  |
| 23 | Input 3 Pulse width LSB |  |
| 24 | Input 3 Pulse width HSB |  |
| 25 | Input 4 Mapping Address |  |
| 26 | Input 4 Mode |  |
| 27 | RFU |  |
| 28 | Input 4 Pulse width LSB |  |
| 29 | Input 4 Pulse width HSB |  |
| 30 | Input 5 Mapping Address |  |
| 31 | Input 5 Mode |  |
| 32 | RFU |  |
| 33 | Input 5 Pulse width LSB |  |
| 34 | Input 5 Pulse width HSB |  |
| 35 | Input 6 Mapping Address |  |
| 36 | Input 6 Mode |  |
| 37 | RFU |  |
| 38 | Input 6 Pulse width LSB |  |
| 39 | Input 6 Pulse width HSB |  |
| 40 | Input 7 Mapping Address |  |
| 41 | Input 7 Mode |  |
| 42 | RFU |  |
| 43 | Input 7 Pulse width LSB |  |
| 44 | Input 7 Pulse width HSB |  |
| 45 | Input 8 Mapping Address |  |
| 46 | Input 8 Mode |  |
| 47 | RFU |  |
| 48 | Input 8 Pulse width LSB |  |
| 49 | Input 8 Pulse width HSB |  |
| 50 | Input 9 Mapping Address |  |
| 51 | Input 9 Mode |  |
| 52 | RFU |  |
| 53 | Input 9 Pulse width LSB |  |
| 54 | Input 9 Pulse width HSB |  |
| 55 | Input 10 Mapping Address |  |
| 56 | Input 10 Mode |  |
| 57 | RFU |  |
| 58 | Input 10 Pulse width LSB |  |
| 59 | Input 10 Pulse width HSB |  |
| 60 | Input 11 Mapping Address |  |
| 61 | Input 11 Mode |  |
| 62 | RFU |  |
| 63 | Input 11 Pulse width LSB |  |
| 64 | Input 11 Pulse width HSB |  |
| 65 | Input 12 Mapping Address |  |
| 66 | Input 12 Mode |  |
| 67 | RFU |  |
| 68 | Input 12 Pulse width LSB |  |
| 69 | Input 12 Pulse width HSB |  |
| 70 | LRC |  |
| 71 | Footer | Fixed BBh |

## Input to Output Triggers (1Dh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 1Dh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Action 1 Input Number (see list here) |  |
| 11 | Action 1 Output Number (see list here) |  |
| 12 | Action 1 Relay Action | 0h: Nothing 1h: Activate 2h: Deactivate |
| 13 | Action 1 Alarm | 00h: Disable 01h: Alarm Enable 02h: GPI Reset Enable 03h: Alarm & GPI Reset Enable |
| 14 | Action 1 Scope | 1h : local 2h : global |
| 15 | Action 1 Time LSB | 1d to 65533d : for seconds 65534d : up to valid input 65535d : infinite time |
| 16 | Action 1 Time HSB |  |
| 17 to N - 9 | …. Action 2 to Action 64 |  |
| N - 8 | Action 64 Input Number (see list here) |  |
| N - 7 | Action 64 Output Number (see list here) |  |
| N - 6 | Action 64 Action |  |
| N - 5 | Action 64 Alarm Enable? |  |
| N - 4 | Action 64 Scope | 1h : local 2h : global |
| N - 3 | Action 64 Time LSB |  |
| N - 2 | Action 64 Time HSB |  |
| n - 1 | LRC |  |
| n | Footer | Fixed BBh |

## Event To Output Triggers (1Eh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 1Eh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door Number | 1d to 4d |
| 11 | Action 1 Event (see list here) |  |
| 12 | Action 1 Input Number (see list here) |  |
| 13 | Action 1 Output Number (see list here) |  |
| 14 | Action 1 Relay Action | 0h: Nothing 1h: Open 2h: Close |
| 15 | Action 1 Alarm | 00h: Disable 01h: Alarm Enable |
| 16 | Action 1 Time LSB | 1d to 65533d : for seconds 65534d : up to valid input 65535d : infinite time |
| 17 | Action 1 Time HSB |  |
| 18 to N - 9 | …. Action 2 to Action 64 |  |
| N - 8 | Action 64 Input Number (see list here) |  |
| N - 7 | Action 64 Output Number (see list here) |  |
| N - 6 | Action 64 Action |  |
| N - 5 | Action 64 Alarm Enable? |  |
| N - 4 | Action 64 Photo Capture Enable? |  |
| N - 3 | Action 64 Time LSB |  |
| N - 2 | Action 64 Time HSB |  |
| n - 1 | LRC |  |
| n | Footer | Fixed BBh |

## Buzzer LED Setup (1Fh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 1Fh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Allow Internal Buzzer Pulses | 1d to 255d |
| 11 | Allow Internal Buzzer On LSB | YYh (in milliseconds) |
| 12 | Allow Internal Buzzer On HSB | XXh |
| 13 | Allow Internal Buzzer Off LSB |  |
| 14 | Allow Internal Buzzer Off HSB |  |
| 15 | Deny Internal Buzzer Pulses |  |
| 16 | Deny Internal Buzzer On LSB |  |
| 17 | Deny Internal Buzzer On HSB |  |
| 18 | Deny Internal Buzzer Off LSB |  |
| 19 | Deny Internal Buzzer Off HSB |  |
| 20 | Allow External Buzzer Pulses |  |
| 21 | Allow External Buzzer On LSB |  |
| 22 | Allow External Buzzer On HSB |  |
| 23 | Allow External Buzzer Off LSB |  |
| 24 | Allow External Buzzer Off HSB |  |
| 25 | Deny External Buzzer Pulses |  |
| 26 | Deny External Buzzer On LSB |  |
| 27 | Deny External Buzzer On HSB |  |
| 28 | Deny External Buzzer Off LSB |  |
| 29 | Deny External Buzzer Off HSB |  |
| 30 | Allow Green LED Pulses |  |
| 31 | Allow Green LED On LSB |  |
| 32 | Allow Green LED On HSB |  |
| 33 | Allow Green LED Off LSB |  |
| 34 | Allow Green LED Off HSB |  |
| 35 | Deny Green LED Pulses |  |
| 36 | Deny Green LED On LSB |  |
| 37 | Deny Green LED On HSB |  |
| 38 | Deny Green LED Off LSB |  |
| 39 | Deny Green LED Off HSB |  |
| 40 | Allow RED LED Pulses |  |
| 41 | Allow RED LED On LSB |  |
| 42 | Allow RED LED On HSB |  |
| 43 | Allow RED LED Off LSB |  |
| 44 | Allow RED LED Off HSB |  |
| 45 | Deny RED LED Pulses |  |
| 46 | Deny RED LED On LSB |  |
| 47 | Deny RED LED On HSB |  |
| 48 | Deny RED LED Off LSB |  |
| 49 | Deny RED LED Off HSB |  |
| 50 | LRC |  |
| 51 | Footer | Fixed BBh |

## Display  & Brightness Parameters (20h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 20h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | First Line Message | 00h: Nothing 01h: User Name 02h: Card ID 03h: Employee ID 04h: Custom MSZ |
| 11 | Second Line Message |  |
| 12 | Third Line Message (only for BIOSCRIBE_3S) |  |
| 13 | Fourth Line Message (only for BIOSCRIBE_3S) |  |
| 14 | Clock Face (For Bioscribe 3S ) | 0h: digital 1h: analouge (default for bioscribe 3s) |
| 15 | Hour Format if Digital Face | 00h: 12 Hours format (default) 01h: 24 hour format |
| 16 | Card ID Display format | ‘H’ (48h) : Hexadecimal display ‘D’ (44h): Decimal Display 00h: default |
| 17 | Display Picture (DP) On | 00h: disable 01h: enable |
| 18 to 49 | Access allowed message | Null terminated 32 byte ascii string |
| 50 to 81 | Access denied message | Null terminated 32 byte ascii string |
| 82 to 113 | Global Message | Null terminated 32 byte ascii string |
| 114 | Screen Saver Enable (Bioscribe 3S) | 00h: Disable 01h: Enable |
| 115 | Screen Saver Timeout | 5d to 255d seconds, 0 if disabled |
| 116 | Screen Saver change picture time | 5d to 255d seconds, 0 if disabled |
| 117 | Active brightness level | 50d to 100d |
| 118 | Idle brightness level | 10d to 49d |
| 119 | Active to Idle transition time LSB | DDh (1d to 255d * 1000 milliseconds = AABBCCDDh) |
| 120 | Active to Idle transition time HSB | CCh |
| 121 | Active to Idle transition time USB | BBh |
| 122 | Active to Idle transition time MSB | AAh |
| 123 | Idle to Off transition time LSB | DDh (1d to 255d * 1000 milliseconds = AABBCCDDh) |
| 124 | Idle to Off transition time HSB | CCh |
| 125 | Idle to Off transition time USB | BBh |
| 126 | Idle to Off transition time MSB | AAh |
| 127 | Active to Idle Transition Enable | 0h: disable 1h: enable |
| 128 | Idle to Off Transition Enable | 0h: disable 1h: enable |
| 129 | LRC |  |
| 130 | Footer | Fixed BBh |

## Sync Card ID Format (21h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 21h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Wigand / Clock Data | 1h: Wigand (if 1 then send below bytes) 0h: Clk dta |
| 12 | Format 1 Wigand Bits | 26d to 254d |
| 13 | Format 1 Reader Type | 0h: em card 1h: HID Prox 2h: HID I class 3h: MI fare Reverse 4h: MI fare Forward 5h: others |
| 14 | Format 1 Site Code (SC) Enable / Disable | 0h: disable 1h: enable |
| 15 | Format 1 ID start bit | 1d to 254d (but < end bit) |
| 16 | Format 1 ID end bit | > start bit but <= 254d |
| 17 | Format 1 SC start bit position | Bits ranges except ID start & end bits |
| 18 | Format 1 SC end bit position | Bits ranges except ID start & end bits |
| 19 | Format 1 Direction forward / reverse | 0h: forward 1h: reverse |
| 20 | Format 1 Parity Enable / Disable | 0h: disable 1h: enable |
| 21 | Format 1 No. of Parity | 1d to 6d |
| 22 | Format 1 Parity position (P1) | Parity bit position within bits |
| 23 | Format 1 Parity Type (P1) | 0h: even 1h: odd |
| 24 | Format 1 Parity Calculation start bit (P1) | bit position where to start calculation |
| 25 | Format 1 Parity Calculation end bit (P1) | bit position where to end calculation |
| 26 | Format 1 Parity position (P2) |  |
| 27 | Format 1 Parity Type (P2) |  |
| 28 | Format 1 Parity Calculation start bit (P2) |  |
| 29 | Format 1 Parity Calculation end bit (P2) |  |
| 30 | Format 1 Parity position (P3) |  |
| 31 | Format 1 Parity Type (P3) |  |
| 32 | Format 1 Parity Calculation start bit (P3) |  |
| 33 | Format 1 Parity Calculation end bit (P3) |  |
| 34 | Format 1 Parity position (P4) |  |
| 35 | Format 1 Parity Type (P4) |  |
| 36 | Format 1 Parity Calculation start bit (P4) |  |
| 37 | Format 1 Parity Calculation end bit (P4) |  |
| 38 | Format 1 Parity position (P5) |  |
| 39 | Format 1 Parity Type (P5) |  |
| 40 | Format 1 Parity Calculation start bit (P5) |  |
| 41 | Format 1 Parity Calculation end bit (P5) |  |
| 42 | Format 1 Parity position (P6) |  |
| 43 | Format 1 Parity Type (P6) |  |
| 44 | Format 1 Parity Calculation start bit (P6) |  |
| 45 | Format 1 Parity Calculation end bit (P6) |  |
|  | Same for Format 2 |  |
|  | Same for Format 3 |  |
|  | Same for Format 4 |  |
|  | Same for Format 5 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Offline Firmware Update (Read Firmware Version) (22h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 22h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 to 14 | Model ID | 4 ascii bytes |
| 15 to -- | Model Name | 16 ascii bytes |
|  | Current Firmware Version | 16 ascii bytes |
|  | Current Firmware Date | 16 ascii bytes |
|  | Boot Loader Version | 7 ascii bytes |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Offline Firmware Update (Upload Firmware Info) (23h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 23h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Total Length in Bytes LSB | DDh (if size is AABBCCDDh) |
| 11 | Total Length in Bytes HSB | CCh |
| 12 | Total Length in Bytes USB | BBh |
| 13 | Total Length in Bytes MSB | AAh |
| 14 | Total Segments LSB | DDh (if AABBCCDDh) |
| 15 | Total Segments HSB | CCh |
| 16 | Total Segments USB | BBh |
| 17 | Total Segments MSB | AAh |
| 18 | CRC16 CCSIT LSB | YYh (if CRC is XXYYh) |
| 19 | CRC16 CCSIT HSB | XXh |
| 20 | LRC |  |
| 21 | Footer | Fixed BBh |

## Offline Firmware Update (Upload Firmware Data) (24h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 24h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Segment Number LSB | DDh (if number is AABBCCDDh) |
| 11 | Segment Number HSB | CCh |
| 12 | Segment Number USB | BBh |
| 13 | Segment Number MSB | AAh |
| 14 | Segment Data Length LSB | DDh (if length is AABBCCDDh) |
| 15 | Segment Data Length HSB | CCh |
| 16 | Segment Data Length USB | BBh |
| 17 | Segment Data Length MSB | AAh |
| 18 to N - 2 | Data bytes (max 1024) | Actual Firmware Data (ascii bytes) |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Offline Firmware Update (Verify & Start upgrade) (25h)

This command will Reset Device if previous commands are executed in proper sequence.  If any error was there this command will report error

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 25h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Factory Reset (26h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 26h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Soft Reset (Restart Device) (27h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 27h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Over Ride Outputs (28h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h / 01h |
| 6 | FC | 28h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Relay 1 Mode BST/BSC/UST = LOCK | 0h: armed 1h: open 2h: closed |
| 11 | Relay 1 Time LSB | 1d to 65533d : for seconds 65535d : infinite time |
| 12 | Relay 1 Time HSB |  |
| 13 | Relay 2 Mode BST/BSC/UST = GPO1 |  |
| 14 | Relay 2 Time LSB |  |
| 15 | Relay 2 Time HSB |  |
| 16 | Relay 3 Mode |  |
| 17 | Relay 3 Time LSB |  |
| 18 | Relay 3 Time HSB |  |
| 19 | Relay 4 Mode |  |
| 20 | Relay 4 Time LSB |  |
| 21 | Relay 5 Time HSB |  |
| 22 | Relay 5 Mode |  |
| 23 | Relay 5 Time LSB |  |
| 24 | Relay 5 Time HSB |  |
| 25 | Relay 6 Mode |  |
| 26 | Relay 6 Time LSB |  |
| 27 | Relay 6 Time HSB |  |
| 28 | Relay 7 Mode |  |
| 29 | Relay 7 Time LSB |  |
| 30 | Relay 7 Time HSB |  |
| 31 | Relay 8 Mode |  |
| 32 | Relay 8 Time LSB |  |
| 33 | Relay 8 Time HSB |  |
| 34 | LRC |  |
| 35 | Footer | Fixed BBh |

## Mi-fare Scan CSN (29h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 29h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 04h |
| 9 | Payload length (HSB) | 00h |
| 10 | Returned Card ID LSB |  |
| 11 | Returned Card ID HSB |  |
| 12 | Returned Card ID USB |  |
| 13 | Returned Card ID MSB |  |
| 14 | LRC |  |
| 15 | Footer | Fixed BBh |

## Mi-fare Write Card (2Ah)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Ah |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Old key byte 1 |  |
| 11 | Old key byte 2 |  |
| 12 | Old key byte 3 |  |
| 13 | Old key byte 4 |  |
| 14 | Old key byte 5 |  |
| 15 | Old key byte 6 |  |
| 16 | Old Key Type | AAh or BBh |
| 17 | New key byte 1 |  |
| 18 | New key byte 2 |  |
| 19 | New key byte 3 |  |
| 20 | New key byte 4 |  |
| 21 | New key byte 5 |  |
| 22 | New key byte 6 |  |
| 23 | New Key Type |  |
| 24 | Block Number | 1d to 255d |
| 25 to 40 | Block Data | 16 bytes of data |
| 41 | LRC |  |
| 42 | Footer | Fixed BBh |

## Mi-fare Release CSN (2Bh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Bh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | LRC |  |
| 11 | Footer | Fixed BBh |

## Mi-fare Sector Mode Config (2Ch)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Index/Block number | See Note |
| 11 | Block Byte 1Byte |  |
| 12 | Block Byte 2Byte |  |
| 13 | Block Byte 3Byte |  |
| 14 | Block Byte 4Byte |  |
| 15 | Block Byte 5Byte |  |
| 16 | Block Byte 6Byte |  |
| 17 | Block Byte 7Byte |  |
| 18 | Block Byte 8Byte |  |
| 19 | Block Byte 9Byte |  |
| 20 | Block Byte 10Byte |  |
| 21 | Block Byte 11Byte |  |
| 22 | Block Byte 12Byte |  |
| 23 | Block Byte 13Byte |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

### Note For Command (2Ch) :-

#### 1.Sector Mode configuration

| Index/Block number | (2d) |
| --- | --- |
| Block Byte 1Byte | RFU |
| Block Byte 2Byte | Reader type CSN / Sector-- 1/2 |

#### Employ Id configuration

| Index/Block number | (4d) |
| --- | --- |
| Block Byte 1Byte | Employee ID On/Of(1/0) |
| Block Byte 2Byte | Employee Sector |
| Block Byte 3Byte | Employee Block |
| Block Byte 4Byte | Employee Byte Address |
| Block Byte 5Byte | Employee Key Type |
| Block Byte 6Byte | Employee Key Byte 1 |
| Block Byte /Byte | Employee Key Byte 2 |
| Block Byte 8Byte | Employee Key Byte 3 |
| Block Byte 9Byte | Employee Key Byte 4 |
| Block Byte 10Byte | Employee Key Byte 5 |
| Block Byte 11Byte | Employee Key Byte 6 |
| Block Byte 12Byte | Employee ID length |
| Block Byte 13Byte | Employee ID format |

#### Name1 configuration

| Index/Block number | (5d) |
| --- | --- |
| Block Byte 1Byte | Name1 On/Of(1/0) |
| Block Byte 2Byte | Name1 Sector |
| Block Byte 3Byte | Name1 Block |
| Block Byte 4Byte | Name1 Byte Address |
| Block Byte 5Byte | Name1 Key Type |
| Block Byte 6Byte | Name1 Key Byte 1 |
| Block Byte /Byte | Name1 Key Byte 2 |
| Block Byte 8Byte | Name1 Key Byte 3 |
| Block Byte 9Byte | Name1 Key Byte 4 |
| Block Byte 10Byte | Name1 Key Byte 5 |
| Block Byte 11Byte | Name1 Key Byte 6 |
| Block Byte 12Byte | Name1 ID length |

#### Name2  configuration

| Index/Block number | (6d) |
| --- | --- |
| Block Byte 1Byte | Name2 On/Of(1/0) |
| Block Byte 2Byte | Name2 Sector |
| Block Byte 3Byte | Name2 Block |
| Block Byte 4Byte | Name2 Byte Address |
| Block Byte 5Byte | Name2 Key Type |
| Block Byte 6Byte | Name2 Key Byte 1 |
| Block Byte /Byte | Name2 Key Byte 2 |
| Block Byte 8Byte | Name2 Key Byte 3 |
| Block Byte 9Byte | Name2 Key Byte 4 |
| Block Byte 10Byte | Name2 Key Byte 5 |
| Block Byte 11Byte | Name2 Key Byte 6 |
| Block Byte 12Byte | Name2 ID length |

#### FP1  configuration

| Index/Block number | (8D) |
| --- | --- |
| Block Byte 1Byte | FP1 On/Of(1/0) |
| Block Byte 2Byte | FP1 Sector |
| Block Byte 3Byte | FP1 Block |
| Block Byte 4Byte | FP1 Byte Address |
| Block Byte 5Byte | FP1 Key Type |
| Block Byte 6Byte | FP1 Key Byte 1 |
| Block Byte /Byte | FP1 Key Byte 2 |
| Block Byte 8Byte | FP1 Key Byte 3 |
| Block Byte 9Byte | FP1 Key Byte 4 |
| Block Byte 10Byte | FP1 Key Byte 5 |
| Block Byte 11Byte | FP1 Key Byte 6 |
| Block Byte 12Byte | Fp Size Low Byte (0x01) |
| Block Byte 13Byte | Fp Size Low Byte (0x40/0x00/0x80) |

#### FP2  configuration

| Index/Block number | (12d) |
| --- | --- |
| Block Byte 1Byte | FP2 On/Of(1/0) |
| Block Byte 2Byte | FP2 Sector |
| Block Byte 3Byte | FP2 Block |
| Block Byte 4Byte | FP2 Byte Address |
| Block Byte 5Byte | FP2 Key Type |
| Block Byte 6Byte | FP2 Key Byte 1 |
| Block Byte /Byte | FP2 Key Byte 2 |
| Block Byte 8Byte | FP2 Key Byte 3 |
| Block Byte 9Byte | FP2 Key Byte 4 |
| Block Byte 10Byte | FP2 Key Byte 5 |
| Block Byte 11Byte | FP2 Key Byte 6 |
| Block Byte 12Byte | Fp Size Low Byte (0x01) |
| Block Byte 13Byte | Fp Size Low Byte (0x40/0x00/0x80) |

#### DOE  configuration

| Index/Block number | (10d) |
| --- | --- |
| Block Byte 1Byte | DOE On/Of(1/0) |
| Block Byte 2Byte | DOE Sector |
| Block Byte 3Byte | DOE Block |
| Block Byte 4Byte | DOE Byte Address |
| Block Byte 5Byte | DOE Key Type |
| Block Byte 6Byte | DOE Key Byte 1 |
| Block Byte /Byte | DOE Key Byte 2 |
| Block Byte 8Byte | DOE Key Byte 3 |
| Block Byte 9Byte | DOE Key Byte 4 |
| Block Byte 10Byte | DOE Key Byte 5 |
| Block Byte 11Byte | DOE Key Byte 6 |
| Block Byte 12Byte | DOE Length |
| Block Byte 13Byte | DOE Format(0/1/2 ) |

#### Transaction on card  configuration

| Index/Block number | (14d) |
| --- | --- |
| Block Byte 1Byte | On/Of(1/0) |
| Block Byte 2Byte | Sector |
| Block Byte 3Byte | Block |
| Block Byte 4Byte | Byte Address |
| Block Byte 5Byte | Key Type |
| Block Byte 6Byte | Key Byte 1 |
| Block Byte /Byte | Key Byte 2 |
| Block Byte 8Byte | Key Byte 3 |
| Block Byte 9Byte | Key Byte 4 |
| Block Byte 10Byte | Key Byte 5 |
| Block Byte 11Byte | Key Byte 6 |
| Block Byte 12Byte | Length |
| Block Byte 13Byte | Total Trans (max 100) |

#### Access Code setting (

| Index/Block number | (21d) |
| --- | --- |
| Block Byte 1Byte | On/Off(1/0) |
| Block Byte 2Byte | Sector |
| Block Byte 3Byte | Block |
| Block Byte 4Byte | Byte Address |
| Block Byte 5Byte | Key Type |
| Block Byte 6Byte | Key Byte 1 |
| Block Byte /Byte | Key Byte 2 |
| Block Byte 8Byte | Key Byte 3 |
| Block Byte 9Byte | Key Byte 4 |
| Block Byte 10Byte | Key Byte 5 |
| Block Byte 11Byte | Key Byte 6 |
| Block Byte 12Byte | Length |
| Block Byte 13Byte | Format ( Hex:0/ Ascii:1/Decimal:2) |

## Sync Reason (2Dh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Dh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Feature Enable Disable | 00h: disable 01h: enable |
| 11 | Function Mode | 00h: optional , enables menu icon on home screen 01h: Mandatory mode, removes icon form home screen |
| 12 | RFU | 0h |
| 13 |  |  |
| 14 | Reason 1 Number | 1d to 255d |
| 15 | Reason 1 Inout (RFU for now) | 00h |
| 16 | Reason 1 RFU | 00h |
| 17 TO 32 | Reason 1 Description | 16 char ascii string |
| 33 | Reason 1 null | 00h |
| 34 | Reason 1 Card ID LSB |  |
| 35 | Reason 1 Card ID HSB |  |
| 36 | Reason 1 Card ID USB |  |
| 37 | Reason 1 Card ID MSB |  |
| 38 | Reason 1 Card EXTD ID LSB (for 64 bit CardID only) |  |
| 39 | Reason 1 Card EXTD ID HSB |  |
| 40 | Reason 1 Card EXTD ID USB |  |
| 41 | Reason 1 Card EXTD ID MSB |  |
| 42 to 69 | Repeat from 13 to 40 for Reason 2 (28 bytes) |  |
| 71 to 97 | Repeat from 13 to 40 for Reason 3 (28 bytes) |  |
| 98 to 125 | Repeat from 13 to 40 for Reason 4 (28 bytes) |  |
| 126 to 153 | Repeat from 13 to 40 for Reason 5 (28 bytes) |  |
| 154 to 181 | Repeat from 13 to 40 for Reason 6 (28 bytes) |  |
| 182 to 209 | Repeat from 13 to 40 for Reason 7 (28 bytes) |  |
| 210 to 237 | Repeat from 13 to 40 for Reason 8 (28 bytes) |  |
| 238 | LRC |  |
| 239 | Footer | Fixed BBh |

## Pause / Resume Transactions (2Eh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Eh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 01h |
| 9 | Payload length (HSB) | 00h |
| 10 | Action | 0h: resume 1h: pause |
| 11 | LRC |  |
| 12 | Footer | Fixed BBh |

## Start Transaction Recovery (2Fh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 2Fh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Unique Sequence Number LSB | 4byte unique id |
| 11 | Unique Sequence Number HSB |  |
| 12 | Unique Sequence Number USB |  |
| 13 | Unique Sequence Number MSB |  |
| 14 | Start Year | 00d to 99d (for 2000 to 2099) |
| 15 | Start Month | 01d to 12d |
| 16 | Start Date | 01d to 31d (keep validation for leap) |
| 17 | Start Hour | 00d to 23d |
| 18 | Start Minute | 00d to 59d |
| 19 | Start Second | 00d to 59d |
| 20 | End Year | 00d to 99d (for 2000 to 2099) |
| 21 | End Month | 01d to 12d |
| 22 | End Date | 01d to 31d (keep validation for leap) |
| 23 | End Hour | 00d to 23d |
| 24 | End Minute | 00d to 59d |
| 25 | End Second | 00d to 59d |
| 26 | LRC |  |
| 27 | Footer | Fixed BBh |

## Clear Transactions (30h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 30h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | CID LSB |  |
| 11 | CID HSB |  |
| 12 | MAC ID 1 |  |
| 13 | MAC ID 2 |  |
| 14 | MAC ID 3 |  |
| 15 | MAC ID 4 |  |
| 16 | MAC ID 5 |  |
| 17 | MAC ID 6 |  |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

## Delete All Rules / Specific Rule(31h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 31h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 02h |
| 9 | Payload length (HSB) | 00h |
| 10 | Rule Number LSB | YYh (XXYYh = Rule Number) (send FFFFh to delete all rules) |
| 11 | Rule Number HSB | XXh |
| 12 | LRC |  |
| 13 | Footer | Fixed BBh |

## Reset Rule State (32h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 32h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Scope | 0h: All Users 1h: Particular User |
| 11 | Rule Number LSB | Rule Number to Reset |
| 12 | Rule Number MSB |  |
| 13 | UID LSB |  |
| 14 | UID HSB |  |
| 15 | UID USB |  |
| 16 | UID MSB |  |
| 17 | EXTD ID LSB (for 64 bit CardID only) |  |
| 18 | EXTD ID HSB |  |
| 19 | EXTD ID USB |  |
| 20 | EXTD ID MSB |  |
| 21 | LRC |  |
| 22 | Footer | Fixed BBh |

## Master Slave Setup (33h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 32h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Device Mode | 0h: Master Mode 1h: Slave Mode |
| 11 | Master IPv4 address segment 1 | C0h (if ip is 192.100.100.90) |
| 12 | Master IPv4 address segment 2 | 64h |
| 13 | Master IPv4 address segment 3 | 64h |
| 14 | Master IPv4 address segment 4 | 5Ah |
| 15 | Master Port LSB |  |
| 16 | Master Port HSB |  |
| 17 | RFU |  |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

## Rule Setup (Normal / Simple APB) (34h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 34h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard 2h : soft |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard 2h : soft |
| 14 | Daily Reset Enable | 0h: Disable 1h: Enable |
| 15 | Daily Reset Time Hour | 0d to 23d |
| 16 | Daily Reset Time Minute | 0d to 59d |
| 17 | Local / Global | 0h: Local Individual Door Wise rule 1h: Global Group Wise Rule (MS Required) |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 21 | User Type USB |  |
| 22 | User Type MSB |  |
| 23 | Door 1 | 0h: Disable on this door 1h: Enable on this door |
| 24 | Door 2 |  |
| 25 | Door 3 |  |
| 26 | Door 4 |  |
| 27 | LRC |  |
| 28 | Footer | Fixed BBh |

## Rule Setup (Timed APB) (35h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 35h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard 2h : soft |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard 2h : soft |
| 14 | Daily Reset Enable | 0h: Disable 1h: Enable |
| 15 | Daily Reset Time Hour | 0d to 23d |
| 16 | Daily Reset Time Minute | 0d to 59d |
| 17 | Local / Global | 0h: Local Individual Door Wise rule 1h: Global Group Wise Rule (MS Required) |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 | 0h: Disable on this door 1h: Enable on this door |
| 23 | Door 2 |  |
| 24 | Door 3 |  |
| 25 | Door 4 |  |
| 26 | Entry To Entry Time LSB | DDh (0d to 86400d = AABBCCDDh) |
| 27 | Entry To Entry Time HSB | CCh |
| 28 | Entry To Entry Time USB | BBh |
| 29 | Entry To Entry Time MSB | AAh |
| 30 | Entry To Exit Time LSB | DDh (0d to 86400d = AABBCCDDh) |
| 31 | Entry To Exit Time HSB | CCh |
| 32 | Entry To Exit Time USB | BBh |
| 33 | Entry To Exit Time MSB | AAh |
| 34 | Exit To Entry Time LSB | DDh (0d to 86400d = AABBCCDDh) |
| 35 | Exit To Entry Time HSB | CCh |
| 36 | Exit To Entry Time USB | BBh |
| 37 | Exit To Entry Time MSB | AAh |
| 38 | Exit To Exit Time LSB | DDh (0d to 86400d = AABBCCDDh) |
| 39 | Exit To Exit Time HSB | CCh |
| 40 | Exit To Exit Time USB | BBh |
| 41 | Exit To Exit Time MSB | AAh |
| 42 | LRC |  |
| 43 | Footer | Fixed BBh |

## Rule Setup (Conditional APB) (36h)

Not Implemented. RFU

## Rule Setup (Concentric Circle) (37h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 37h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard 2h : soft |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard 2h : soft |
| 14 | Daily Reset Enable | 0h: Disable 1h: Reset on Specified Time 2h: Reset on GPI Trigger 3h: Reset on Time or GPI |
| 15 | Daily Reset Time Hour | 0d to 23d |
| 16 | Daily Reset Time Minute | 0d to 59d |
| 17 | Reset GPI Number | 1h to 12h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 layer Number | 0h: rule does not apply on this door 1h, 2h, 3h layer number |
| 23 | Door 2 layer Number |  |
| 24 | Door 3 layer Number |  |
| 25 | Door 4 layer Number |  |
| 26 | LRC |  |
| 27 | Footer | Fixed BBh |

## Rule Setup (Matrix APB) (38h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 38h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Entry Strictness | 1h : hard 2h : soft |
| 13 | Exit Strictness | 1h : hard 2h : soft |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | RFU | 0h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 Group Number | 0d: rule does not apply on this door 1d to 8d: group number 255d: this is a reset door |
| 23 | Door 2 Group Number |  |
| 24 | Door 3 Group Number |  |
| 25 | Door 4 Group Number |  |
| 26 | Rule Reset After Minutes LSB | 0d: reset functionality disabled 1d to 65535d minutes |
| 27 | Rule Reset After Minutes HSB |  |
| 28 | Minimum Time In Reset Gate LSB | 0d: reset functionality disabled 1d to 65535d minutes |
| 29 | Minimum Time In Reset Gate HSB |  |
| 30 | Row 1 Column1 access (1st cell on table) | 0bAABBCCDD (1 byte) where AA = Exit to Entry access BB = Entry to Exit access (RFU) CC = Exit to Exit access. (RFU) DD = Entry to Entry access.(RFU) Possible values. 00b: NULL 01b: Allow 10b: Deny |
| 31 | Row 1 Column2 access (2nd cell on table) |  |
| 32 to … | Cell 3rd to cell 63 access |  |
| N - 2 | Row 8 Column8 access (64th cell on table) |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Rule Setup (Access Limit) (39h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 39h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 1h : hard 2h : soft |
| 13 | RFU | 0h |
| 14 | Daily Reset Enable | 0h: Disable 1h: Enable |
| 15 | Daily Reset Time Hour | 0d to 23d |
| 16 | Daily Reset Time Minute | 0d to 59d |
| 17 | Local / Global | 0h: Local Individual Door Wise rule 1h: Global Group Wise Rule (MS Required) |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 21 | User Type USB |  |
| 22 | User Type MSB |  |
| 23 | Door 1 | 0h: disable 1h: enable |
| 24 | Door 2 |  |
| 25 | Door 3 |  |
| 26 | Door 4 |  |
| 27 | Access Limit LSB | 0h: Disable on this door 1h to 65000h limit |
| 28 | Access Limit HSB |  |
| 29 | LRC |  |
| 30 | Footer | Fixed BBh |

## Rule Setup (Absent User) (3Ah)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Ah |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h: disabled 1h : hard 2h : soft |
| 13 | RFU | 0h |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | Local / Global | 0h: Local Individual Door Wise rule 1h: Global Group Wise Rule (MS Required) |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 | 0h: disable 1h: enable |
| 23 | Door 2 |  |
| 24 | Door 3 |  |
| 25 | Door 4 |  |
| 26 | Absent Days | 1d to 250d |
| 27 | LRC |  |
| 28 | Footer | Fixed BBh |

## Rule Setup (Custodian) (3Bh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Bh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h: disabled 1h : hard 2h : soft |
| 13 | RFU | 0h |
| 14 | Daily Reset Enable | 0h: Disable 1h: Enable |
| 15 | Daily Reset Time Hour | 0d to 23d |
| 16 | Daily Reset Time Minute | 0d to 59d |
| 17 | RFU | 0h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 | 0h: disable 1h: enable |
| 23 | Door 2 |  |
| 24 | Door 3 |  |
| 25 | Door 4 |  |
| 26 | Custodian 1 UID LSB | DDh (If card id is AABBCCDDh) |
| 27 | Custodian 1 UID HSB | CCh |
| 28 | Custodian 1 UID USB | BBh |
| 29 | Custodian 1 UID MSB | AAh |
| 30 | Custodian 1 EXTD ID LSB (for 64 bit CardID only) | DDh (If card id is AABBCCDDh) |
| 31 | Custodian 1 EXTD ID HSB | CCh |
| 32 | Custodian 1 EXTD ID USB | BBh |
| 33 | Custodian 1 EXTD ID MSB | AAh |
| 34 | Custodian 2 UID LSB |  |
| 35 | Custodian 2 UID HSB |  |
| 36 | Custodian 2 UID USB |  |
| 37 | Custodian 2 UID MSB |  |
| 38 | Custodian 2 EXTD ID LSB (for 64 bit CardID only) |  |
| 39 | Custodian 2 EXTD ID HSB |  |
| 40 | Custodian 2 EXTD ID USB |  |
| 41 | Custodian 2 EXTD ID MSB |  |
| 42 | Custodian 3 UID LSB |  |
| 45 | Custodian 3 UID HSB |  |
| 46 | Custodian 3 UID USB |  |
| 47 | Custodian 3 UID MSB |  |
| 48 | Custodian 3 EXTD ID LSB (for 64 bit CardID only) |  |
| 49 | Custodian 3 EXTD ID HSB |  |
| 50 | Custodian 3 EXTD ID USB |  |
| 51 | Custodian 3 EXTD ID MSB |  |
| 52 | Custodian 4 UID LSB |  |
| 53 | Custodian 4 UID HSB |  |
| 54 | Custodian 4 UID USB |  |
| 55 | Custodian 4 UID MSB |  |
| 56 | Custodian 4 EXTD ID LSB (for 64 bit CardID only) |  |
| 57 | Custodian 4 EXTD ID HSB |  |
| 58 | Custodian 4 EXTD ID USB |  |
| 59 | Custodian 4 EXTD ID USB |  |
| 60 | Custodian 5 UID LSB |  |
| 61 | Custodian 5 UID HSB |  |
| 62 | Custodian 5 UID USB |  |
| 63 | Custodian 5 UID MSB |  |
| 64 | Custodian 5 EXTD ID LSB (for 64 bit CardID only) |  |
| 65 | Custodian 5 EXTD ID HSB |  |
| 66 | Custodian 5 EXTD ID USB |  |
| 67 | Custodian 5 EXTD ID MSB |  |
| 68 | LRC |  |
| 69 | Footer | Fixed BBh |

## Rule Setup (Multiple Occupancy) (3Ch)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard 2h : soft |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard 2h : soft |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | Local / Global | 0h: Local Individual Door Wise rule 1h: Global Group Wise Rule (MS Required) |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 | 0h: disable 1h: enable |
| 23 | Door 2 |  |
| 24 | Door 3 |  |
| 25 | Door 4 |  |
| 26 | Maximum Count LSB | YYh (0d to 65000d) (0d for no limit) |
| 27 | Maximum Count HSB | XXh |
| 28 | Minimum Count LSB | 0d to 255d (0d for no limit) |
| 29 | LRC |  |
| 30 | Footer | Fixed BBh |

## Rule Setup (Sentry Check) (3Dh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Dh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | RFU | 0h |
| 13 | RFU | 0h |
| 14 | RFU | 0h |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 18 | User Type HSB |  |
| 19 | User Type USB |  |
| 20 | User Type MSB |  |
| 21 | Door 1 | 0h: disable 1h: enable |
| 22 | Door 2 |  |
| 23 | Door 3 |  |
| 24 | Door 4 |  |
| 25 | Period Minutes (LSB) | YYh (where XXYYd = 1d to 1440d minutes) |
| 26 | Period Minutes (HSB) | XXh |
| 27 | Access Type | 1h: allow access 2h: do not allow access |
| 28 | Minimum Man Alive | 1d to 5d |
| 29 | Dead Man 1 UID LSB |  |
| 30 | Dead Man 1 UID HSB |  |
| 31 | Dead Man 1 UID USB |  |
| 32 | Dead Man 1 UID MSB |  |
| 33 | Dead Man 1 EXTD ID LSB (for 64 bit CardID only) |  |
| 34 | Dead Man 1 EXTD ID HSB |  |
| 35 | Dead Man 1 EXTD ID USB |  |
| 36 | Dead Man 1 EXTD ID MSB |  |
| 37 | Dead Man 2 UID LSB |  |
| 38 | Dead Man 2 UID HSB |  |
| 39 | Dead Man 2 UID USB |  |
| 40 | Dead Man 2 UID MSB |  |
| 41 | Dead Man 2 EXTD ID LSB (for 64 bit CardID only) |  |
| 42 | Dead Man 2 EXTD ID HSB |  |
| 43 | Dead Man 2 EXTD ID USB |  |
| 44 | Dead Man 2 EXTD ID MSB |  |
| 45 | Dead Man 3 UID LSB |  |
| 46 | Dead Man 3 UID HSB |  |
| 47 | Dead Man 3 UID USB |  |
| 48 | Dead Man 3 UID MSB |  |
| 49 | Dead Man 3 EXTD ID LSB (for 64 bit CardID only) |  |
| 50 | Dead Man 3 EXTD ID HSB |  |
| 51 | Dead Man 3 EXTD ID USB |  |
| 52 | Dead Man 3 EXTD ID MSB |  |
| 53 | Dead Man 4 UID LSB |  |
| 54 | Dead Man 4 UID HSB |  |
| 55 | Dead Man 4 UID USB |  |
| 56 | Dead Man 4 UID MSB |  |
| 57 | Dead Man 4 EXTD ID LSB (for 64 bit CardID only) |  |
| 58 | Dead Man 4 EXTD ID HSB |  |
| 59 | Dead Man 4 EXTD ID USB |  |
| 60 | Dead Man 4 EXTD ID MSB |  |
| 61 | Dead Man 5 UID LSB |  |
| 62 | Dead Man 5 UID HSB |  |
| 63 | Dead Man 5 UID USB |  |
| 64 | Dead Man 5 UID MSB |  |
| 65 | Dead Man 5 EXTD ID LSB (for 64 bit CardID only) |  |
| 66 | Dead Man 5 EXTD ID HSB |  |
| 67 | Dead Man 5 EXTD ID USB |  |
| 68 | Dead Man 5 EXTD ID MSB |  |
| 69 | LRC |  |
| 70 | Footer | Fixed BBh |

## Rule Setup (Web Post) (3Eh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Eh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard 2h : soft |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard 2h : soft |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | RFU | 1h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Payload format | 0h: HTTP Post (JSON) 1h: ASCII (JSON only, no http header) 2h: Raw Binary |
| 23 | Online Verification Needed? | 0h: no 1h: yes (server should response with “True”/”False” ASCII string case sensitive. In case of RAW format 1h/0h |
| 24 | Server IPv4 Segment 1 |  |
| 25 | Server IPv4 Segment 2 |  |
| 26 | Server IPv4 Segment 3 |  |
| 27 | Server IPv4 Segment 4 |  |
| 28 | Server Port LSB |  |
| 29 | Server Port HSB |  |
| 30 | SSL enable on this port | 0h: disable 1h: enable |
| 31 | Info Field LSB | Bit wise or ed together 00000000000000000000000000000001b: Card ID 00000000000000000000000000000010b: Site Code 00000000000000000000000000000100b: CID 00000000000000000000000000001000b: Date Time 00000000000000000000000000010000b: Reader Number 00000000000000000000000000100000b: Gate Number 00000000000000000000000001000000b: Access Info |
| 32 | Info Field HSB |  |
| 33 | Info Field USB |  |
| 34 | Info Field MSB |  |
| 35 to N - 2 | HTTP api path | 255d ASCII string zero padded |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Rule Setup (Interlocking) (3Fh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 3Fh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | RFU | 0h |
| 13 | RFU | 0h |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | No Of Doors In this Rule | 1h to 8h |
| 18 | RFU | 0h |
| 19 | RFU |  |
| 20 | RFU |  |
| 21 | With Switch / Switch Less | Bit wise 0b : With Switch / 1b : Switch Less Example: ABCDEFGHb where (1 byte ) A : Door 8 B : Door 7 C : Door 6 D : Door 5 E : Door 4 F : Door 3 G : Door 2 H : Door 1 |
| 22 | Door 1 Index | 0h : disable 1h to 8h : valid Indies from DIT matrix |
| 23 | Door 2 Index |  |
| 24 | Door 3 Index |  |
| 25 | Door 4 Index |  |
| 26 | Other Door State On Door 1 Open | Bit wise + or ed together Valid values for each bit 0b : unlocked 1b: locked Example: ABCDEFGHb where (1 byte ) A : Door 8 B : Door 7 C : Door 6 D : Door 5 E : Door 4 F : Door 3 G : Door 2 H : Door 1 |
| 27 | Other Door State On Door 2 Open |  |
| 28 | Other Door State On Door 3 Open |  |
| 29 | Other Door State On Door 4 Open |  |
| 30 | Other Door State On Door 5 Open |  |
| 31 | Other Door State On Door 6 Open |  |
| 32 | Other Door State On Door 7 Open |  |
| 33 | Other Door State On Door 8 Open |  |
| 34 | Delay after Door 1 Close | Bit wise + or ed together Valid values for each bit 0b : no 1b: yes Example: ABCDEFGHb where (1 byte ) A : Door 8 B : Door 7 C : Door 6 D : Door 5 E : Door 4 F : Door 3 G : Door 2 H : Door 1 |
| 35 | Delay after Door 2 Close |  |
| 36 | Delay after Door 3 Close |  |
| 37 | Delay after Door 4 Close |  |
| 38 | Delay after Door 5 Close |  |
| 39 | Delay after Door 6 Close |  |
| 40 | Delay after Door 7 Close |  |
| 41 | Delay after Door 8 Close |  |
| 42 | Buzzer / LED Indication While Door 1 Open | Bit wise + or ed together Valid values for each bit 0b : no 1b: yes Example: ABCDEFGHb where (1 byte ) A : Door 8 B : Door 7 C : Door 6 D : Door 5 E : Door 4 F : Door 3 G : Door 2 H : Door 1 |
| 43 | Buzzer / LED Indication While Door 2 Open |  |
| 44 | Buzzer / LED Indication While Door 3 Open |  |
| 45 | Buzzer / LED Indication While Door 4 Open |  |
| 46 | Buzzer / LED Indication While Door 5 Open |  |
| 47 | Buzzer / LED Indication While Door 6 Open |  |
| 48 | Buzzer / LED Indication While Door 7 Open |  |
| 49 | Buzzer / LED Indication While Door 8 Open |  |
| 50 | Door 1 Delay Time LSB | YYh (where YYXXh ranges from 0d to 65000d) |
| 51 | Door 1 Delay Time HSB | XXh |
| 52 | Door 2 Delay Time LSB |  |
| 53 | Door 2 Delay Time HSB |  |
| 54 | Door 3 Delay Time LSB |  |
| 55 | Door 3 Delay Time HSB |  |
| 56 | Door 4 Delay Time LSB |  |
| 57 | Door 4 Delay Time HSB |  |
| 58 | Door 5 Delay Time LSB |  |
| 59 | Door 5 Delay Time HSB |  |
| 60 | Door 6 Delay Time LSB |  |
| 61 | Door 6 Delay Time HSB |  |
| 62 | Door 7 Delay Time LSB |  |
| 63 | Door 7 Delay Time HSB |  |
| 64 | Door 8 Delay Time LSB |  |
| 65 | Door 8 Delay Time HSB |  |
| 66 | Fail Safe / Fail Secure? | 0h: Fail Secure mechanism 1h: Fail Safe Mechanism |
| 67 | Fail Safe Time LSB | YYh (where YYXXh ranges from 0d to 65000d) |
| 68 | Fail Safe Time HSB | XXh |
| 69 | LRC |  |
| 70 | Footer | Fixed BBh |

## Rule Setup (Group Authentication) (40h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 40h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | RFU | 0h |
| 17 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 18 | User Type HSB |  |
| 19 | User Type USB |  |
| 20 | User Type MSB |  |
| 21 | Door 1 Entry Authentication mode | 0h: Single / Do not apply 1h: Dual Authentication 2h: Triple Authentication |
| 22 | Door 1 Exit Authentication |  |
| 23 | Door 2 Entry Authentication |  |
| 24 | Door 2 Exit Authentication |  |
| 25 | Door 3 Entry Authentication |  |
| 26 | Door 3 Exit Authentication |  |
| 27 | Door 4 Entry Authentication |  |
| 28 | Door 4 Exit Authentication |  |
| 29 | Group Enable/Disable | Enable: 0x01 / Disable: 0x00 |
| 30 | Door 1 Entry Group No1 LSB |  |
| 31 | Door 1 Entry Group No1 HSB |  |
| 32 | Door 1 Entry Group No2 LSB |  |
| 33 | Door 1 Entry Group No2 HSB |  |
| 34 | Door 1 Entry Group No3 LSB |  |
| 35 | Door 1 Entry Group No3 HSB |  |
| 36 | Door 1 Exit Group No1 LSB |  |
| 37 | Door 1 Exit Group No1 HSB |  |
| 38 | Door 1 Exit Group No2 LSB |  |
| 39 | Door 1 Exit Group No2 HSB |  |
| 40 | Door 1 Exit Group No3 LSB |  |
| 41 | Door 1 Exit Group No3 HSB |  |
| 42 | Door 2 Entry Group No1 LSB |  |
| 43 | Door 2 Entry Group No1 HSB |  |
| 44 | Door 2 Entry Group No2 LSB |  |
| 45 | Door 2 Entry Group No2 HSB |  |
| 46 | Door 2 Entry Group No3 LSB |  |
| 47 | Door 2 Entry Group No3 HSB |  |
| 48 | Door 2 Exit Group No1 LSB |  |
| 49 | Door 2 Exit Group No1 HSB |  |
| 50 | Door 2 Exit Group No2 LSB |  |
| 51 | Door 2 Exit Group No2 HSB |  |
| 52 | Door 2 Exit Group No3 LSB |  |
| 53 | Door 2 Exit Group No3 HSB |  |
| 54 | Door 3 Entry Group No1 LSB |  |
| 55 | Door 3 Entry Group No1 HSB |  |
| 56 | Door 3 Entry Group No2 LSB |  |
| 57 | Door 3 Entry Group No2 HSB |  |
| 58 | Door 3 Entry Group No3 LSB |  |
| 59 | Door 3 Entry Group No3 HSB |  |
| 60 | Door 3 Exit Group No1 LSB |  |
| 61 | Door 3 Exit Group No1 HSB |  |
| 62 | Door 3 Exit Group No2 LSB |  |
| 63 | Door 3 Exit Group No2 HSB |  |
| 64 | Door 3 Exit Group No3 LSB |  |
| 65 | Door 3 Exit Group No3 HSB |  |
| 66 | Door 4 Entry Group No1 LSB |  |
| 67 | Door 4 Entry Group No1 HSB |  |
| 68 | Door 4 Entry Group No2 LSB |  |
| 69 | Door 4 Entry Group No2 HSB |  |
| 70 | Door 4 Entry Group No3 LSB |  |
| 71 | Door 4 Entry Group No3 HSB |  |
| 72 | Door 4 Exit Group No1 LSB |  |
| 73 | Door 4 Exit Group No1 HSB |  |
| 74 | Door 4 Exit Group No2 LSB |  |
| 75 | Door 4 Exit Group No2 HSB |  |
| 76 | Door 4 Exit Group No3 LSB |  |
| 77 | Door 4 Exit Group No3 HSB |  |
| 78 | LRC |  |
| 79 | Footer | Fixed BBh |

## Rule Setup (Multiple Authentication) (40h)	(Deprecated, Do not use)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 40h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 0h : disabled 1h : hard |
| 13 | Rule Strictness Exit | 0h : disabled 1h : hard |
| 14 | RFU | 0h |
| 14 | RFU | 0h |
| 15 | RFU | 0h |
| 16 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 17 | User Type HSB |  |
| 18 | User Type USB |  |
| 19 | User Type MSB |  |
| 20 | Door 1 Entry Authentication mode | 0h: Single 1h: Double 2h: Triple |
| 21 | Door 1 Exit Authentication |  |
| 22 | Door 2 Entry Authentication |  |
| 23 | Door 2 Exit Authentication |  |
| 24 | Door 3 Entry Authentication |  |
| 25 | Door 3 Exit Authentication |  |
| 26 | Door 4 Entry Authentication |  |
| 27 | Door 4 Exit Authentication |  |
| 28 | LRC |  |
| 29 | Footer | Fixed BBh |

## Get finger Template (41h)

Read mode will get only local templates (from sd card), will not get templates from fp module. Returned data length will depend upon finger size setting in fppara. (nominally 384 bytes per finger template)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | 0Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Number Of Templates Required | 1h or 2h |
| 11 | UID LSB |  |
| 12 | UID HSB |  |
| 13 | UID USB |  |
| 14 | UID MSB |  |
| 15 | EXTD ID LSB (for 64 bit CardID only) |  |
| 16 | EXTD ID HSB |  |
| 17 | EXTD ID USB |  |
| 18 | EXTD ID MSB |  |
| 19 | Template 1 Index | 0h to 9h |
| 20 | Template 2 Index | 0h to 9h |
| 21 | Returned Template 1 size LSB | 80h (if 384d = 180h) |
| 22 | Returned Template 1 size HSB | 01h |
| 23.. to M | Returned Template 1 Data byte 0 |  |
|  | … |  |
| M | Returned Template 1 Data byte last |  |
| M+1 | Returned Template 2 size LSB |  |
| M+2 | Returned Template 2 size HSB |  |
| M+3 | Returned Template 2 Data byte 0 |  |
| … | … |  |
| N - 2 | Returned Template 2 Data byte last |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Delete Specific Parameters (42h)

This command may take time if multiple parameters are being deleted

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 42h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Delete All Users + User Exceptions + Fingers | 0h: no 1h: yes |
| 11 | Delete All Access Tables (TZ, Wekkoff, Holiday, TZ grp) |  |
| 12 | Delete Other Para (GPI, Door Exceptions, Door mode, Input Actions, Event Actions, Rules, APB) |  |
| 13 | RFU |  |
| 14 | RFU |  |
| 15 | RFU |  |
| 16 | RFU |  |
| 17 | RFU |  |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

## User Anti pass-back Override (43h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 43h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 18h |
| 9 | Payload length (HSB) | 00h |
| 10 | UID LSB |  |
| 11 | UID HSB |  |
| 12 | UID USB |  |
| 13 | UID MSB |  |
| 14 | Site Code LSB |  |
| 15 | Site Code HSB |  |
| 16 | Site Code USB |  |
| 17 | Site Code MSB |  |
| 18 | Set or Delete on Door 1 | 0h: no change 1h: set 2h: delete |
| 19 | Set or Delete on Door 2 |  |
| 20 | Set or Delete on Door 3 |  |
| 21 | Set or Delete on Door 4 |  |
|  | What to Override | 1h: ALL APB 2h: Access Limit 4h: Absent user 8h: Custodian 10h: Occupancy 20h: Multiple Auth |
| 22 | RFU | 0h |
| 23 | LRC |  |
| 24 | Footer | Fixed BBh |

## User Authentication change / Finger Bypass (44h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 44h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 09h |
| 9 | Payload length (HSB) | 00h |
| 10 | UID LSB |  |
| 11 | UID HSB |  |
| 12 | UID USB |  |
| 13 | UID MSB |  |
| 14 | Site Code LSB |  |
| 15 | Site Code HSB |  |
| 16 | Site Code USB |  |
| 17 | Site Code MSB |  |
| 18 | Enable Bypass on Door 1 | 1h: enable 0h: disable |
| 19 | Enable Bypass on Door 2 |  |
| 20 | Enable Bypass on Door 3 |  |
| 21 | Enable Bypass on Door 4 |  |
| 22 | Entry Authentication Mode Door 1 | 01h: Card Only 02h: Card + Pin 03h: Card / Pin 04h: Card + Finger 05h: Card + Pin + Finger 06h: Only Finger 07h: Card / Finger |
| 23 | Exit Authentication Mode Door 1 |  |
| 24 | Entry Authentication Mode Door 2 |  |
| 25 | Exit Authentication Mode Door 2 |  |
| 26 | Entry Authentication Mode Door 3 |  |
| 27 | Exit Authentication Mode Door 3 |  |
| 28 | Entry Authentication Mode Door 4 |  |
| 29 | Exit Authentication Mode Door 4 |  |
| 30 | LRC |  |
| 31 | Footer | Fixed BBh |

## User Custom Message (45h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 45h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 09h |
| 9 | Payload length (HSB) | 00h |
| 10 | UID LSB |  |
| 11 | UID HSB |  |
| 12 | UID USB |  |
| 13 | UID MSB |  |
| 14 | Site Code LSB |  |
| 15 | Site Code HSB |  |
| 16 | Site Code USB |  |
| 17 | Site Code MSB |  |
| 18 | Start Year |  |
| 19 | Start Month |  |
| 20 | Start Date |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | End Year |  |
| 24 | End Month |  |
| 25 | End Data |  |
| 26 | RFU |  |
| 27 | RFU |  |
| 28 | Custom Message | 32 characters ascii string |
| 29 | LRC |  |
| 30 | Footer | Fixed BBh |

## NTP Parameters (46h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 46h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | NTP client on off | 0h: disabled 1h: enabled |
| 11 | Time zone Sign | 1d: negative 2d: positive |
| 12 | Time zone hour | 0d to 12d |
| 13 | Time zone minute | 0d to 59d |
| 14 | NTP server address | 32 char ascii string. Must be valid web address excluding ‘Http://’ or ‘Https://’ |
| 14 + 32 | LRC |  |
| 14 + 33 | Footer | Fixed BBh |

## Camera capture on software events (47h) (variable length command)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 46h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Capture with GPI 1 alarm | 0h: no 1h: yes |
| 11 | Capture with GPI 2 alarm |  |
| 12 | RFU |  |
| 13 | RFU |  |
| 14 | Capture with Temper 1 Alarm |  |
| 15 | Capture with Temper 2 Alarm |  |
| 16 | Capture with This Event 1 | Software Event code. See Event list |
| 17 | Capture with This Event 2 |  |
| 18 | Capture with This Event 3 |  |
| 19 | Capture with This Event … |  |
| … | … |  |
| N - 2 | …. |  |
| N -1 | LRC |  |
| N | Footer | Fixed BBh |

## Device Password Reset (48h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 48h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 09h |
| 9 | Payload length (HSB) | 00h |
| 10 | Pass Char 1 | Ascii string max 8 char |
| 11 | Pass Char 2 | Alpha numeric for BIOSCRIBE 3S |
| 12 | Pass Char 3 | Numeric only for TwinXS 3S |
| 13 | Pass Char 4 |  |
| 14 | Pass Char 5 |  |
| 15 | Pass Char 6 |  |
| 16 | Pass Char 7 |  |
| 17 | Pass Char 8 |  |
| 18 | Blank Byte | 00h |
| 19 | Admin Card ID 1 LSB | DDh (if id is AABBCCDDh) |
| 20 | Admin Card ID 1 HSB | CCh |
| 21 | Admin Card ID 1 USB | BBh |
| 22 | Admin Card ID 1 MSB | AAh |
| 23 | Admin Card ID 2 LSB |  |
| 24 | Admin Card ID 2 HSB |  |
| 25 | Admin Card ID 2 USB |  |
| 26 | Admin Card ID 2 MSB |  |
| 27 | Admin Card ID 3 LSB |  |
| 28 | Admin Card ID 3 HSB |  |
| 29 | Admin Card ID 3 USB |  |
| 30 | Admin Card ID 3 MSB |  |
| 31 | Admin Card ID 4 LSB |  |
| 32 | Admin Card ID 4 HSB |  |
| 33 | Admin Card ID 4 USB |  |
| 34 | Admin Card ID 4 MSB |  |
| 35 | Admin Card ID 5 LSB |  |
| 36 | Admin Card ID 5 HSB |  |
| 37 | Admin Card ID 5 USB |  |
| 38 | Admin Card ID 5 MSB |  |
| 39 | LRC |  |
| 40 | Footer | Fixed BBh |

## BLE Para (49h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 49h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | BLE access enable? | 00h: disabled 01h: enabled |
| 11 | Tx Power Level | 0d to 127d |
| 12 | Key Byte 1 |  |
| 13 | Key Byte 2 |  |
| 14 | Key Byte 3 |  |
| 15 | Key Byte 4 |  |
| 16 | Key Byte 5 |  |
| 17 | Key Byte 6 |  |
| 18 to 41 | Door Name | 24 byte ascii string.. if name is shorter than 24 bytes pad with hex 00h |
| 42 | LRC |  |
| 43 | Footer | Fixed BBh |

## Set Finger Encryption (4Ah)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 2Dh |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Ah |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 21h |
| 9 | Payload length (HSB) | 00h |
| 10 | Fingerprint template encryption enable? | 00h: disabled 01h: enabled |
| 11 to 43 | 32 byte AES Key | 32 bytes key |
| 44 | LRC |  |
| 45 | Footer | Fixed BBh |

## Add Extended User Information (4Bh)

Note: This command may be expanded in future to incorporate more information.

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Bh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | UID LSB | DDh (if card id is AABBCCDDh) |
| 11 | UID HSB | CCh |
| 12 | UID USB | BBh |
| 13 | UID MSB | AAh |
| 14 | EXTD ID LSB (for 64 bit CardID only) | DDh (if card id is AABBCCDDh) |
| 15 | EXTD ID HSB | CCh |
| 16 | EXTD ID USB | BBh |
| 17 | EXTD ID MSB | AAh |
| 18 | Update Escort | UpdateEscort: 0x01 otherwise 0x00 |
| 19 | Escort group ID LSB Door 1 | 2 Byte group ID for Escort Rule, numeric or alpha numeric |
| 20 | Escort group ID HSB Door 1 |  |
| 21 | Escort group ID LSB Door 2 |  |
| 22 | Escort group ID HSB Door 2 |  |
| 23 | Escort group ID LSB Door 3 |  |
| 24 | Escort group ID HSB Door 3 |  |
| 25 | Escort group ID LSB Door 4 |  |
| 26 | Escort group ID HSB Door 4 |  |
| 27 | Update Multiple Auth group | Update Multiple Auth group: 0x01 otherwise 0x00 |
| 28 | Multiple Auth group1 ID LSB | 2 Byte group ID for Multiple Auth Rule |
| 29 | Multiple Auth group1 ID HSB |  |
| 30 | Multiple Auth group2 ID LSB |  |
| 31 | Multiple Auth group2 ID HSB |  |
| 32 | Multiple Auth group3 ID LSB |  |
| 33 | Multiple Auth group3 ID HSB |  |
| 34 | Multiple Auth group4 ID LSB |  |
| 35 | Multiple Auth group4 ID HSB |  |
| 36 | LRC |  |
| 37 | Footer | Fixed BBh |

## Enable / Disable Ethernet Modules (F9h)

Upon executing this command in write mode device will reboot.

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 11h |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | F9h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 05h |
| 9 | Payload length (HSB) | 00h |
| 10 | FTP module | 00h: Disable 01h: Enable |
| 11 | HTTP web module |  |
| 12 | TCP Modbus Module |  |
| 13 | SNMP Module |  |
| 14 | BLE Module |  |
| 15 | LRC |  |
| 16 | Footer | Fixed BBh |

## Sound & Volume Parameters (4Ch)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 10h |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Ch |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 04h |
| 9 | Payload length (HSB) | 00h |
| 10 | Key Volume | 0d to 10 d |
| 11 | Notification Volume |  |
| 12 | Allow volume style | 01h: Beep style (for allow it will play 1 long beep, for deny it will play 3 short beep) 02h: Voice prompt |
| 13 | Deny volume style |  |
| 14 | LRC |  |
| 15 | Footer | Fixed BBh |

## User password (Pin) Change (4Dh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 19h |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Dh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 0Dh |
| 9 | Payload length (HSB) | 00h |
| 10 | UID LSB | 78h(If UID is 0x12345678) |
| 11 | UID HSB | 56h |
| 12 | UID USB | 34h |
| 13 | UID MSB | 12h |
| 14 | EXTD ID LSB (for 64 bit CardID only) | 78h(If EXTD ID is 0x12345678) (Currently all zeros) |
| 15 | EXTD ID HSB | 56h |
| 16 | EXTD ID USB | 34h |
| 17 | EXTD ID MSB | 12h |
| 18 | PIN LSB | 00h (set all zeros in this field to reset pin, else set pin) |
| 19 | PIN HSB | 00h |
| 20 | PIN USB | 00h |
| 21 | PIN MSB | 00h |
| 22 | Door id | 01h, 02h, 03h or 04h |
| 23 | LRC |  |
| 24 | Footer | Fixed BBh |

## Rule Setup (Escort) (4Eh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 78h |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Eh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 6Ch |
| 9 | Payload length (HSB) | 00h |
| 10 | Rule Number LSB | YYh (if rule number is XXYYh) |
| 11 | Rule Number HSB | XXh |
| 12 | RFU | 00h |
| 13 | RFU | 00h |
| 14 | RFU | 00h |
| 15 | RFU | 00h |
| 16 | RFU | 00h |
| 17 | RFU | 00h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Table 1 Doors to be covered in this rule | Bit wise encoded + or ed together 00000001b: for door 1 00000010b: for door 2 00000100b: for door 3 00001000b: for door 4 |
| 23 | Table 1 Mandatory? | 01h: Escort is required all the time 02h: Conditional Escort |
| 24 | Table 1 Condition: TimeZone, Holiday, Weekend, ETC. | 01h: Use This condition. 02h: Do not use this condition. |
| 25 | Table 1 Condition: FingerPrint Auth |  |
| 26 | Table 1 Condition: APB |  |
| 27 | Table 1 RFU1 |  |
| 28 | Table 1 RFU2 |  |
| 29 | Table 1 RFU3 |  |
| 30 | Table 1 Escort User Type LSB | 2 Byte code for Escort, either numeric or alpha numeric. |
| 31 | Table 1 Escort User Type HSB |  |
| 32 | Table 1 Escortee User Type LSB | 2 Byte code for Escortee, either numeric or alpha numeric. |
| 33 | Table 1 Escortee User Type HSB |  |
| 34 to 45 | Repeat 22 to 33 for Table 2 | Same as Table 1 |
| 46 to 57 | Repeat 22 to 33 for Table 3 |  |
| 58 to 69 | Repeat 22 to 33 for Table 4 |  |
| 70 to 81 | Repeat 22 to 33 for Table 5 |  |
| 82 to 93 | Repeat 22 to 33 for Table 6 |  |
| 94 to 105 | Repeat 22 to 33 for Table 7 |  |
| 106 to 117 | Repeat 22 to 33 for Table 8 |  |
| 118 | LRC |  |
| 119 | Footer | Fixed BBh |

## Rule Setup (MQTT Publisher) (4Fh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 00h |
| 2 | Total Length (HSB) | 02h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 4Fh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | F4h |
| 9 | Payload length (HSB) | 01h |
| 10 | Rule Number LSB | YYh (if rule number is XXYYh) |
| 11 | Rule Number HSB | XXh |
| 12 | RFU | 00h |
| 13 | RFU | 00h |
| 14 | RFU | 00h |
| 15 | RFU | 00h |
| 16 | RFU | 00h |
| 17 | RFU | 00h |
| 18 | User Type LSB | Bit wise encoded + or ed together 00000000000000000000000000000001b: for type 1 10000000000000000000000000000000b: for type 32 |
| 19 | User Type HSB |  |
| 20 | User Type USB |  |
| 21 | User Type MSB |  |
| 22 | Door 1 Enable | 00h: Disable 01: Enable |
| 23 | Door 2 Enable |  |
| 24 | Door 3 Enable |  |
| 25 | Door 4 Enable |  |
| 26 | SSL Enabled |  |
| 27 | QOS (RFU) | 00h |
| 28 | PORT LSB | YYh (if port number is XXYYh) |
| 29 | PORT MSB | XXh |
| 30 to 125 | Server Name | Null terminated ASCII chars (96 MAX) |
| 126 to 221 | Publish Topic Name | Null terminated ASCII chars (96 MAX) |
| 222 to 245 | MQTT User Name | Null terminated ASCII chars (24 MAX) |
| 246 to 269 | MQTT User Password | Null terminated ASCII chars (24 MAX) |
| 270 to 293 | MQTT Client ID | Null terminated ASCII chars (24 MAX) |
| 294 | Field 1 Code | 00h: Field Disable 01h: Card ID 02h: Employee Name 03h: Employee ID 04h: CID 05h: Date & Time 06h: Access Code 07h: In / Out 08h: Reader Number 09h: Keypad Input |
| 295 to 309 | Field 1 Name | Null terminated ASCII chars (14 MAX) |
| 310 | Field 1 Null | 00h |
| 311 | Field 2 Code |  |
| 312 to 325 | Field 2 Name |  |
| 326 | Field 2 Null |  |
| 327 | Field 3 Code |  |
| 328 to 341 | Field 3 Name |  |
| 342 | Field 3 Null |  |
| 343 | Field 4 Code |  |
| And so on | Field 4 Name |  |
|  | Field 4 Null |  |
|  | Field 5 Code |  |
|  | Field 5 Name |  |
|  | Field 5 Null |  |
|  | Field 6 Code |  |
|  | Field 6 Name |  |
|  | Field 6 Null |  |
|  | Field 7 Code |  |
|  | Field 7 Name |  |
|  | Field 7 Null |  |
|  | Field 8 Code |  |
|  | Field 8 Name |  |
|  | Field 8 Null |  |
| 422 | Keypad Input Enable | 00h: disable 01h: Enable |
| 423 | Keypad Input Type | 00h: Numeric 01h: Alpha Numeric |
| 424 | Keypad Input length | 1d to 8d |
| 425 | RFU | 00h |
| 426 | RFU | 00h |
| 427 | RFU | 00h |
| 428 | RFU | 00h |
| 429 | RFU | 00h |
| 430 to 517 | RFU | 00h |
| 518 | LRC |  |
| 519 | Footer | Fixed BBh |

## Temperature Detection parameters (50h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 1Ch |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 50h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 10h |
| 9 | Payload length (HSB) | 00h |
| 10 | Temperature Reader Enabled | 0h: No 01h: Yes |
| 11 | Temperature Threshold Integer part | 25h (37d.80d = 25h.50h) (37.80 °Celsius is default) |
| 12 | Temperature Threshold Fraction Part | 50h |
| 13 | Temperature Threshold Check | 00h: Disable 01h: Enable |
| 14 | RFU | 00h |
| 15 | RFU | 00h |
| 16 | RFU | 00h |
| 17 | RFU | 00h |
| 18 | RFU | 00h |
| 19 | RFU | 00h |
| 20 | RFU | 00h |
| 21 | RFU | 00h |
| 22 | RFU | 00h |
| 23 | RFU | 00h |
| 24 | RFU | 00h |
| 25 | RFU | 00h |
| 26 | LRC |  |
| 27 | Footer | Fixed BBh |

## Arm disarm mode settings (51h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 10h |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 51h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 04h |
| 9 | Payload length (HSB) | 00h |
| 10 | DOOR 1 | 00h: Door Armed 01h: Door Disarmed |
| 11 | DOOR 2 |  |
| 12 | DOOR 3 |  |
| 13 | DOOR 4 |  |
| 14 | LRC |  |
| 15 | Footer | Fixed BBh |

## Mifare MADConfig (52h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 52h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 11 | Application Index | 1d to 8d |
| 12 | This is MAD Config | 00h (Fix) |
| 13 | Enable | 00h: Disable 01h: Enable |
| 14 | App ID LSB | YYh (if APP id is XXYYh) |
| 15 | App ID MSB | XXh |
| 16 | MAD Key Type | AAh or BBh |
| 17 | MAD Key Byte 1 |  |
| 18 | MAD Key Byte 2 |  |
| 19 | MAD Key Byte 3 |  |
| 20 | MAD Key Byte 4 |  |
| 21 | MAD Key Byte 5 |  |
| 22 | MAD Key Byte 6 |  |
| 16 | App Key Type | AAh or BBh |
| 17 | App Key Byte 1 |  |
| 18 | App Key Byte 2 |  |
| 19 | App Key Byte 3 |  |
| 20 | App Key Byte 4 |  |
| 21 | App Key Byte 5 |  |
| 22 | App Key Byte 6 |  |
| 23 | Skip Validation? | 00h: No 01h: Yes |
| 24 | LRC |  |
| 25 | Footer | Fixed BBh |

## Mifare MAD Parameters(52h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 52h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 11 | Application Index | 1d to 8d |
| 12 | MAD Parameters | 01h: Access Code 02h: Employee Name 03h: Employee ID 04h: Date of Start 05h: Date of Expiry 06h: FP1 07h: FP2 |
| 13 | Enable? | 00h: disabled 01h: Enabled |
| 14 | Data Offset LSB | YYh (if Offset = XXYYh) |
| 15 | Data Offset MSB | XXh |
| 16 | Data Length LSB | YYh (if Length = XXYYh) |
| 17 | Data Length MSB | XXh |
| 18 | Data Format | 00h: Hex Format 01h: ASCII Format 02h: BCD Format |
| 19 | LRC |  |
| 20 | Footer | Fixed BBh |

## OSDP Controller Setup(53h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 53h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | OSDP Enable | 1: OSDP Enable 0: OSDP disable |
| 11 | Port LSB | Lower byte of port number |
| 12 | Port MSB | Higher byte of port number |
| 13 | RFU |  |
| 14 | RFU |  |
| 15 | RFU |  |
| 16 | RFU |  |
| 17 | RFU |  |
| 18 | RFU |  |
| 19 | Reader 1 type | 0:disable 1: RS485 2 : IP/Ethernet |
| 20 | RFU |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | Reader IPv4 address segment 1 | 00 in case of RS485 otherwise IP address |
| 24 | Reader IPv4 address segment 2 | 00 in case of RS485 otherwise IP address |
| 25 | Reader IPv4 address segment 3 | 00 in case of RS485 otherwise IP address |
| 26 | Reader IPv4 address segment 4 | 00 in case of RS485 otherwise IP address |
| 27 | Reader 2 type | 0:disable 1: RS485 2 : IP/Ethernet |
| 28 | RFU |  |
| 29 | RFU |  |
| 30 | RFU |  |
| 31 | Reader IPv4 address segment 1 | 00 in case of RS485 otherwise IP address |
| 32 | Reader IPv4 address segment 2 | 00 in case of RS485 otherwise IP address |
| 33 | Reader IPv4 address segment 3 | 00 in case of RS485 otherwise IP address |
| 34 | Reader IPv4 address segment 4 | 00 in case of RS485 otherwise IP address |
| 35 | Reader 3 type | 0:disable 1: RS485 2 : IP/Ethernet |
| 36 | rfu |  |
| 37 | RFU |  |
| 38 | RFU |  |
| 39 | Reader IPv4 address segment 1 | 00 in case of RS485 otherwise IP address |
| 40 | Reader IPv4 address segment 2 | 00 in case of RS485 otherwise IP address |
| 41 | Reader IPv4 address segment 3 | 00 in case of RS485 otherwise IP address |
| 42 | Reader IPv4 address segment 4 | 00 in case of RS485 otherwise IP address |
| 43 | Reader 4 type | 0:disable 1: RS485 2 : IP/Ethernet |
| 44 | RFU |  |
| 45 | RFU |  |
| 46 | RFU |  |
| 47 | Reader IPv4 address segment 1 |  |
| 48 | Reader IPv4 address segment 2 |  |
| 49 | Reader IPv4 address segment 3 |  |
| 50 | Reader IPv4 address segment 4 |  |
| 51 | LRC |  |
| 53 | Footer | Fixed BBh |

## Osdp reader sETUP(54h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 54h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | OSDP Enable | 0h:OSDP mode disable 1h:OSDP mode enable |
| 11 | Master IP address byte 1 |  |
| 12 | Master IP address byte 2 |  |
| 13 | Master IP address byte 3 |  |
| 14 | Master IP address byte 4 |  |
| 15 | Port LSB |  |
| 16 | Port MSB |  |
| 17 | Address (1 to 4 ) , Reader ID | 1 to 4 |
| 18 | isFailSafe | 0h: Fail Secure 1h: Fail Safe |
| 19 | RFU |  |
| 20 | RFU |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | LRC |  |
| 24 | Footer | Fixed BBh |

## EAC Reader setup(55h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | 55h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Reader Number(1 to 4) | XXh |
| 11 | ELSGW IPv4 address segment 1 | C0h (if ip is 192.100.100.90) |
| 12 | ELSGW IPv4 address segment 2 | 64h |
| 13 | ELSGW IPv4 address segment 3 | 64h |
| 14 | ELSGW IPv4 address segment 4 | 5Ah |
| 15 | Multicast IPv4 address segment 1 | C0h (if ip is 192.100.100.101) |
| 16 | Multicast IPv4 address segment 2 | 64h |
| 17 | Multicast IPv4 address segment 3 | 64h |
| 18 | Multicast IPv4 address segment 4 | 65h |
| 19 | Port Number LSB | YYh |
| 20 | Port Number MSB | XXh |
| 21 | ELSGW Address LSB | XXh |
| 22 | Receiver Address LSB | XXh |
| 23 | Verification Type LSB | XXh |
| 24 | Verification Location LSB | XXh |
| 25 | Boarding Floor LSB | XXh |
| 26 | Boarding Floor HSB | YYh |
| 27 | Boarding F/R LSB | XXh |
| 28 | Destination F/R LSB | XXh |
| 29 | Terminal Number LSB | If (Terminal number is 0x1234 then ) 0x12 |
| 30 | Terminal Number HSB | 0x34 |
| 31 | LRC |  |
| 32 | Footer | Fixed BBh |

## Rule Setup Server Auth(56h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 56h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Rule Number (LSB) | YYh (1d to 65000d) |
| 11 | Rule Number (HSB) | XXh |
| 12 | Rule Strictness Entry | 1h : hard 2h : soft |
| 13 | Rule Strictness Exit |  |
| 14 | RFU |  |
| 15 | RFU |  |
| 16 | RFU |  |
| 17 | RFU |  |
| 18 | RFU |  |
| 19 | RFU |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | Door 1 | 0h: disable 1h: enable |
| 24 | Door 2 |  |
| 25 | Door 3 |  |
| 26 | Door 4 |  |
| 27 | Apply rule on Entry | 0h: no 1h: yes |
| 28 | Apply rule on Exit |  |
| 29 | Skip User Search |  |
| 30 | Skip User Validation |  |
| 31 | RFU |  |
| 32 | RFU |  |
| 33 | RFU |  |
| 34 | RFU |  |
| 29 | LRC |  |
| 30 | Footer | Fixed BBh |

## Event Action Extended(57h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 57h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door Number | 1h to 4h |
| 11 | Event 1 Delay LSB | XXYYh = 1d to 65533d where |
| 12 | Event 1 Delay HSB |  |
| 13 | Event 1 RFU |  |
| 14 | Event 1 RFU |  |
| 15 | Event 1 RFU |  |
| 16 | Event 1 RFU |  |
| 17 | Event 1 RFU |  |
| 18 + 7 | Same for Event 2 |  |
|  | Repeat 11 to 17 for 64 times |  |
| N-2 | Same for Event 64 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Input Action Extended(58h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | 58h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Door Number | 1h to 4h |
| 11 | Event 1 Delay LSB | XXYYh = 1d to 65533d where |
| 12 | Event 1 Delay HSB |  |
| 13 | Event 1 RFU |  |
| 14 | Event 1 RFU |  |
| 15 | Event 1 RFU |  |
| 16 | Event 1 RFU |  |
| 17 | Event 1 RFU |  |
| 18 + 7 | Same for Event 2 |  |
|  | Repeat 11 to 17 for 64 times |  |
| N-2 | Same for Event 64 |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Confifurable output control (59h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h / 01h |
| 6 | FC | 59h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Relay 1 Mode BSC/UST = LOCK | 0h: Normally ON 1h: Normally OFF |
| 11 | Relay 1 Time LSB | 1 ~ 1000S LSB |
| 12 | Relay 1 Time HSB | 1 ~ 1000S HSB |
| 13 | Relay 2 Mode BSC/UST = GPO1 | 0h: Normally ON 1h: Normally OFF |
| 14 | Relay 2 Time LSB | 1 ~ 1000S LSB |
| 15 | Relay 2 Time HSB | 1 ~ 1000S HSB |
| 16 | RFU |  |
| 17 | RFU |  |
| 18 | RFU |  |
| 19 | RFU |  |
| 20 | RFU |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | RFU |  |
| 24 | RFU |  |
| 25 | RFU |  |
| 26 | RFU |  |
| 27 | RFU |  |
| 28 | RFU |  |
| 29 | RFU |  |
| 30 | LRC |  |
| 31 | FOOTER | BBh |

## Server Auth Query(D1h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | D1h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | QueryID LSB | Per device Unique ID for each query. |
| 11 | QueryID HLSB |  |
| 12 | QueryID USB |  |
| 13 | QueryID MSB |  |
| 14 | Rule number LSB |  |
| 15 | Rule number HSB |  |
| 16 | Door ID | 1d to 4d |
| 17 | InOut | 0h: null 1h: in 2h: out |
| 18 | CardID LSB |  |
| 19 | CardID HLSB |  |
| 21 | CardID USB |  |
| 22 | CardID MSB |  |
| 23 | Extd CardID LSB |  |
| 24 | Extd CardID HLSB |  |
| 25 | Extd CardID USB |  |
| 26 | Extd CardID MSB |  |
| 27 | Hour |  |
| 28 | Minute |  |
| 29 | Second |  |
| 30 | Date |  |
| 31 | Month |  |
| 32 | Year |  |
| 33 | LRC |  |
| 34 | Footer | Fixed BBh |

## Server Auth Response(D1h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 02h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | D1h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | QueryID LSB | Per device Unique ID for each query. |
| 11 | QueryID HLSB |  |
| 12 | QueryID USB |  |
| 13 | QueryID MSB |  |
| 14 | AllowDenyNoSay | 0h: no say, continue as per device auth 1h: forcefully allow 2h: deny |
| 15 |  |  |
| 16 | Deny Reason | 16 byte ascii string |
| 17 | RFU | 0h |
| 18 | LRC |  |
| 19 | Footer | Fixed BBh |

# Canteen Management System Commands

## Item Database (C0h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C0h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Item Number (LSB) | (1d to 512d) |
| 11 | Item Number (HSB) |  |
| 12 | Active / Inactive | 1 : Active 0 : Inactive |
| 13 | Item Price (LSB) | (1d to 1000000d) If item price is 100 Rs, then price field should be 10000d (2710h) in Paisa. Item Price (LSB) = 10h; Item Price (HSB) = 27h; Item Price (USB) = 00h; Item Price (MSB) = 00h; |
| 14 | Item Price (HSB) |  |
| 15 | Item Price (USB) |  |
| 16 | Item Price (MSB) |  |
| 17 + 16 | Item Name | 16 Bytes in ASCII string. |
| 33 | Week Day Group | Bit wise encoded + or ed together 00000001b: for Sunday 01000000b: for Saturday |
| 34 | Canteen Code |  |
| 35 | Vendor Code | 1d to 127d |
| 36 | LRC |  |
| 37 | Footer | Fixed BBh |

## Item Delete (C1h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C1h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Item Number (LSB) | (1d to 512d) for single item delete FFFFh (65535d) for All items delete |
| 11 | Item Number (HSB) |  |
| 12 | LRC |  |
| 13 | Footer | Fixed BBh |

## Add Item Group(C2h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C2h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Item Group Number | 1d to 99d |
| 11 | Location ID |  |
| 12 | Item Number (LSB) | (1d to 512d) |
| 13 | Item Number (HSB) |  |
| 14 | Timezone1 Number | 1d to 127d |
| 15 | Timezone2 Number | 1d to 127d |
| 16 | Timezone3 Number | 1d to 127d |
| 17 | Timezone4 Number | 1d to 127d |
| 18 | Item Restriction Type | 0 : Unlimited 1 : Timezone wise 2 : Daily 3 : Weekly 4 : Monthly |
| 19 | Item Restriction Quantity | 1d to 99d |
| 20 | Subsidize Price (LSB) | (1d to 1000000d) If subsidize price is 100 Rs, then price field should be 10000d (2710h) in Paisa. |
| 21 | Subsidize Price (HSB) |  |
| 22 | Subsidize Price (USB) |  |
| 23 | Subsidize Price (MSB) |  |
| 24 | Subsidize Restriction Type | 0 : Unlimited 1 : Timezone wise 2 : Daily 3 : Weekly 4 : Monthly |
| 25 | Subsidize Restriction Quantity | 1d to 99d |
| 26 | RFU |  |
| 27 | RFU |  |
| 28 | RFU |  |
| 29 | RFU |  |
| 30 | RFU |  |
| 31 | RFU |  |
| 32 | RFU |  |
| 33 | RFU |  |
| 34 | RFU |  |
| 35 | RFU |  |
| 36 | RFU |  |
| 37 | RFU |  |
| 38 | RFU |  |
| 39 | RFU |  |
| 40 | RFU |  |
| 41 | LRC |  |
| 42 | Footer | Fixed BBh |

## Group Item Delete (C3h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C3h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Item Group Number | 1d to 99d : for delete specific item from Group. 255d : for delete all groups |
| 10 | Item Number (LSB) | (1d to 512d) : for single item delete. FFFFh : for delete all items from specified group. |
| 11 | Item Number (HSB) |  |
| 12 | LRC |  |
| 13 | Footer | Fixed BBh |

## Vendor DB (C4h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C4h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Vendor ID | 0d to 127d |
| 11 to N-2 | Vendor Name | 32 Characters in ASCII string |
| N-1 | LRC |  |
| N | Footer | Fixed BBh |

## Print Slip Configuration (C5h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C5h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Print Slip | 00h : Disable 01h : Enable |
| 11 | Line Feed | 0d to 9d |
| 12 | Auto Cutter Enabled | 0d: Disable 1d: Enable |
| 13 | Auto Cutter Command Byte 1 |  |
| 14 | Auto Cutter Command Byte 2 |  |
| 15 | Print Slip Field 1 | 01h: Company Header 02h: Employee Name 03h: Card ID 04h: Employee ID 05h: Vendor Name 06h: Date & Time 07h: Item & Quantity 08h: Transaction Amount 09h: Remaining Balance 0ah: Previous Balance 0bh: Auxiliary Header1 0ch: Auxiliary Header2 0dh: Canteen Code 0eh: Controller ID |
| 16 | Print Slip Field 2 |  |
| 17 | Print Slip Field 3 |  |
| 18 | Print Slip Field 4 |  |
| 19 | Print Slip Field 5 |  |
| 20 | Print Slip Field 6 |  |
| 21 | Print Slip Field 7 |  |
| 22 | Print Slip Field 8 |  |
| 23 | Print Slip Field 9 |  |
| 24 | Print Slip Field 10 |  |
| 25 | Print Slip Field 11 |  |
| 26 | Print Slip Field 12 |  |
| 27 | Print Slip Field 13 |  |
| 28 | Print Slip Field 14 |  |
| 29 | Print Slip Field 15 |  |
| 30 | LRC |  |
| 31 | Footer | Fixed BBh |

## Canteen Parameters (C6h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C6h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Multiple Item Selection | 0 : Disable 1 : Enable |
| 11 | Multiple Quantity Selection |  |
| 12 | Master Slave Configuration | 0 : Stand Alone 1 : Validation from Server |
| 13 | RFU |  |
| 14 | RFU |  |
| 15 | RFU |  |
| 16 | LRC |  |
| 17 | Footer | Fixed BBh |

## Print Header/Index Data (C7h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C7h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Company Header / Auxiliary Index | 0 : Company Header 1 : Auxiliary Index1 2 : Auxiliary Index2 |
| 11 | Data Byte1 |  |
| … |  |  |
| … |  |  |
| 42 | Data Byte32 |  |
| 43 | LRC |  |
| 44 | Footer | Fixed BBh |

## Query to Master (C8h) ,  Master Slave Communication

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C8h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | Card ID (LSB) |  |
| 11 | Card ID (HSB) |  |
| 12 | Card ID (USB) |  |
| 13 | Card ID (MSB) |  |
| 14 | Extended ID (LSB) |  |
| 15 | Extended ID (HSB) |  |
| 16 | Extended ID (USB) |  |
| 17 | Extended ID (MSB) |  |
| 18 | Current Date |  |
| 19 | Current Month |  |
| 20 | Current Year |  |
| 21 | Current Hour |  |
| 22 | Current Minute |  |
| 23 | Current Second |  |
| 24 | Item1 Number LSB |  |
| 25 | Item1 Number MSB |  |
| 26 | Item1 Quantity |  |
| 27 | Item1 Group Number |  |
| 28 | Item1 Canteen Code |  |
| 29 | Item1 Vendor Code |  |
| 30 | Item2 Number LSB |  |
| 31 | Item2 Number MSB |  |
| 32 | Item2 Quantity |  |
| 33 | Item2 Group Number |  |
| 34 | Item2 Canteen Code |  |
| 35 | Item2 Vendor Code |  |
| 36 | Item3 Number LSB |  |
| 37 | Item3 Number MSB |  |
| 38 | Item3 Quantity |  |
| 39 | Item3 Group Number |  |
| 40 | Item3 Canteen Code |  |
| 41 | Item3 Vendor Code |  |
| 42 | Item4 Number LSB |  |
| 43 | Item4 Number MSB |  |
| 44 | Item4 Quantity |  |
| 45 | Item4 Group Number |  |
| 46 | Item4 Canteen Code |  |
| 47 | Item4 Vendor Code |  |
| 48 | Item5 Number LSB |  |
| 49 | Item5 Number MSB |  |
| 50 | Item5 Quantity |  |
| 51 | Item5 Group Number |  |
| 52 | Item5 Canteen Code |  |
| 53 | Item5 Vendor Code |  |
| 54 | LRC |  |
| 55 | Footer | Fixed BBh |

## Response from Master (C8h) ,  Master Slave Communication

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | C8h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 00h |
| 9 | Payload length (HSB) | 00h |
| 10 | Card ID (LSB) |  |
| 11 | Card ID (HSB) |  |
| 12 | Card ID (USB) |  |
| 13 | Card ID (MSB) |  |
| 14 | Site Code (LSB) |  |
| 15 | Site Code (HSB) |  |
| 16 | Site Code (USB) |  |
| 17 | Site Code (MSB) |  |
| 18 | Item1 Number LSB |  |
| 19 | Item1 Number MSB |  |
| 20 | Item1 Price LSB |  |
| 21 | Item1 Price HSB |  |
| 22 | Item1 Price USB |  |
| 23 | Item1 Price MSB |  |
| 24 | Item1 Quantity |  |
| 25 | Item1 Error Code | 01h : Successful XXh : Error Code (See below Table) |
| 26 | Item2 Number LSB |  |
| 27 | Item2 Number MSB |  |
| 28 | Item2 Price LSB |  |
| 29 | Item2 Price HSB |  |
| 30 | Item2 Price USB |  |
| 31 | Item2 Price MSB |  |
| 32 | Item2 Quantity |  |
| 33 | Item2 Error Code | 01h : Successful XXh : Error Code (See below Table) |
| 34 | Item3 Number LSB |  |
| 35 | Item3 Number MSB |  |
| 36 | Item3 Price LSB |  |
| 37 | Item3 Price HSB |  |
| 38 | Item3 Price USB |  |
| 39 | Item3 Price MSB |  |
| 40 | Item3 Quantity |  |
| 41 | Item3 Error Code | 01h : Successful XXh : Error Code (See below Table) |
| 42 | Item4 Number LSB |  |
| 43 | Item4 Number MSB |  |
| 44 | Item4 Price LSB |  |
| 45 | Item4 Price HSB |  |
| 46 | Item4 Price USB |  |
| 47 | Item4 Price MSB |  |
| 48 | Item4 Quantity |  |
| 49 | Item4 Error Code | 01h : Successful XXh : Error Code (See below Table) |
| 50 | Item5 Number LSB |  |
| 51 | Item5 Number MSB |  |
| 52 | Item5 Price LSB |  |
| 53 | Item5 Price HSB |  |
| 54 | Item5 Price USB |  |
| 55 | Item5 Price MSB |  |
| 56 | Item5 Quantity |  |
| 57 | Item5 Error Code | 01h : Successful XXh : Error Code (See below Table) |
| 58 | Transaction Balance (LSB) |  |
| 69 | Transaction Balance (HSB) |  |
| 60 | Transaction Balance (USB) |  |
| 61 | Transaction Balance (MSB) |  |
| 62 | User Remaining Balance (LSB) |  |
| 63 | User Remaining Balance (HSB) |  |
| 64 | User Remaining Balance (USB) |  |
| 65 | User Remaining Balance (MSB) |  |
| 66 | User Previous Balance (LSB) |  |
| 67 | User Previous Balance (HSB) |  |
| 68 | User Previous Balance (USB) |  |
| 69 | User Previous Balance (MSB) |  |
| 70 | LRC |  |
| 71 | Footer | Fixed BBh |

## Canteen Master/Slave Communication Error Codes

| Byte Index | Data |
| --- | --- |
| 1d | Successful |
| 101d | Daily Item Limit Exceeded |
| 102d | Weekly Item Limit Exceeded |
| 103d | Monthly Item Limit Exceeded |
| 104d | Timezone Wise Item Limit Exceeded |
| 105d | Balance Limit Exceeded |

# Push Command Codes (Device To Service)

## Connection Identifier(E0)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 25h (37d) |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E0h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 19h (25d) |
| 9 | Payload length (HSB) | 00h |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | CID LSB | YYh (if CID is XXYYh) |
| 17 | CID HSB | XXh |
| 18 | Communication Interface | 1h: ETH 2h: WIFI 3h: GSM |
| 19 to 28 | Protocol Version Char 1 to 10 | 16 Char ASCII string |
| 29 | Current Year |  |
| 30 | Current Month |  |
| 31 | Current Date |  |
| 32 | Current Hour |  |
| 33 | Current Minute |  |
| 34 | Current Second |  |
| 35 | LRC |  |
| 36 | Footer | Fixed BBh |

## Push Transactions (E1h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E1h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | No of transactions in this frame | 1d to 32d |
| 17 | T1 UID LSB |  |
| 18 | T1 UID HSB |  |
| 19 | T1 UID USB |  |
| 20 | T1 UID MSB |  |
| 21 | T1 SC LSB |  |
| 22 | T1 SC HSB |  |
| 23 | T1 SC USB |  |
| 24 | T1 SC MSB |  |
| 25 | T1 CID LSB |  |
| 26 | T1 CID HSB |  |
| 27 | T1 Transaction Type | 50h: ‘P’ punch Transaction 48h: ‘H’ hard alarm 53h: ‘S’ soft alarm 47h: ‘G’ gps data 43h: ‘C’ Canteen data |
| 28 | T1 Year | 0dto 99d |
| 29 | T1 Month | 1d to 12d |
| 30 | T1 Date | 1d t o31d |
| 31 | T1 Hour | 0d to 23d |
| 32 | T1 Minute | 0d to 59d |
| 33 | T1 Second | 0d to 59d |
| 34 | T1 Access Info / Input Number | See Access Events / Input numbers |
| 35 | T1 Reader Number | 1d to 8d |
| 36 | T1 Door Number | 1d to 4d |
| 37 | T1 RFU | 0h |
| 38 | T1 In punch / out punch | 0d: null 1d: in 2d: out |
| 39 | T1 Reason Flag | 0h: no reason, any other value (see reason codes) |
| 40 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 41 | T1 Trans ID LSB |  |
| 42 | T1 Trans ID HSB |  |
| 43 | T1 Trans ID USB |  |
| 44 | T1 Trans ID MSB |  |
| 45 to … | T2 data |  |
| … to … | T3 data |  |
| … to N-2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Push Photo Header (E2h) (Deprecated)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E2h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | Image ID LSB |  |
| 17 | Image ID HSB |  |
| 18 | Image ID USB |  |
| 19 | Image ID MSB |  |
| 20 | Image Size in Byte LSB |  |
| 21 | Image Size in Byte HSB |  |
| 22 | Image Size in Byte USB |  |
| 23 | Image Size in Byte MSB |  |
| 24 | Total Chunks LSB |  |
| 25 | Total Chunks HSB |  |
| 26 | Last Chunk Size LSB |  |
| 27 | Last Chunk Size HSB |  |
| 28 | CID LSB |  |
| 29 | CID HSB |  |
| 30 | LRC |  |
| 31 | Footer | Fixed BBh |

## Push Photo Data (E3h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E3h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | CID LSB | YYh (if CID is XXYYh)) |
| 17 | CID HSB | XXh |
| 18 | Image ID LSB | DDh (if AABBCCDDh)) |
| 19 | Image ID HSB | CCh |
| 20 | Image ID USB | BBh |
| 21 | Image ID MSB | AAh |
| 22 | Total Chunks LSB | DDh (if AABBCCDDh)) |
| 23 | Total Chunks HSB | CCh |
| 24 | Total Chunks USB | BBh |
| 25 | Total Chunks MSB | AAh |
| 26 | Current Chunk LSB | DDh (if AABBCCDDh)) |
| 27 | Current Chunk HSB | CCh |
| 28 | Current Chunk USB | BBh |
| 29 | Current Chunk MSB | AAh |
| 30 to N -2 | Image Data | Jpg file data |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Push Hard Alarms (E4h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E4h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | No of transactions in this frame | 1d to 32d |
| 17 | RFU |  |
| 18 | RFU |  |
| 19 | RFU |  |
| 20 | RFU |  |
| 21 | RFU |  |
| 22 | RFU |  |
| 23 | RFU |  |
| 24 | RFU |  |
| 25 | T1 CID LSB |  |
| 26 | T1 CID HSB |  |
| 27 | T1 Transaction Type | 50h: ‘P’ punch Transaction 48h: ‘H’ hard alarm 53h: ‘S’ soft alarm 47h: ‘G’ gps data 43h: ‘C’ Canteen data |
| 28 | T1 Year | 0dto 99d |
| 29 | T1 Month | 1d to 12d |
| 30 | T1 Date | 1d t o31d |
| 31 | T1 Hour | 0d to 23d |
| 32 | T1 Minute | 0d to 59d |
| 33 | T1 Second | 0d to 59d |
| 34 | Input Number | Input numbers |
| 35 | RFU | 1d to 8d |
| 36 | RFU | 1d to 4d |
| 37 | RFU | 1d to 4d |
| 38 | RFU | 0d: in 1d: out |
| 39 | RFU | 0h: no reason, any other value see reason codes |
| 40 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 41 | T1 Trans ID LSB |  |
| 42 | T1 Trans ID HSB |  |
| 43 | T1 Trans ID USB |  |
| 44 | T1 Trans ID MSB |  |
| 45 to … | T2 data |  |
| … to … | T3 data |  |
| … to N-2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Push Soft Alarms (E5h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E5h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | No of transactions in this frame | 1d to 32d |
| 17 | T1 UID LSB |  |
| 18 | T1 UID HSB |  |
| 19 | T1 UID USB |  |
| 20 | T1 UID MSB |  |
| 21 | T1 SC LSB |  |
| 22 | T1 SC HSB |  |
| 23 | T1 SC USB |  |
| 24 | T1 SC MSB |  |
| 25 | T1 CID LSB |  |
| 26 | T1 CID HSB |  |
| 27 | T1 Transaction Type | 50h: ‘P’ punch Transaction 48h: ‘H’ hard alarm 53h: ‘S’ soft alarm 47h: ‘G’ gps data 43h: ‘C’ Canteen data |
| 28 | T1 Year | 0dto 99d |
| 29 | T1 Month | 1d to 12d |
| 30 | T1 Date | 1d t o31d |
| 31 | T1 Hour | 0d to 23d |
| 32 | T1 Minute | 0d to 59d |
| 33 | T1 Second | 0d to 59d |
| 34 | T1 Access Info / Input Number | See Access Events / Input numbers |
| 35 | T1 Reader Number | 1d to 8d |
| 36 | T1 Door Number | 1d to 4d |
| 37 | T1 Area number | 1d to 4d |
| 38 | T1 In punch / out punch | 0d: in 1d: out |
| 39 | T1 Reason Flag | 0h: no reason, any other value see reason codes |
| 40 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 41 | T1 Trans ID LSB |  |
| 42 | T1 Trans ID HSB |  |
| 43 | T1 Trans ID USB |  |
| 44 | T1 Trans ID MSB |  |
| 45 to … | T2 data |  |
| … to … | T3 data |  |
| … to N-2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Push GPS Data (E6h)

## Push Canteen Data (E7h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E7h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | No of transactions in this frame | 1d to 32d |
| 17 | T1 UID LSB |  |
| 18 | T1 UID HSB |  |
| 19 | T1 UID USB |  |
| 20 | T1 UID MSB |  |
| 21 | T1 SC LSB |  |
| 22 | T1 SC HSB |  |
| 23 | T1 SC USB |  |
| 24 | T1 SC MSB |  |
| 25 | T1 CID LSB |  |
| 26 | T1 CID HSB |  |
| 27 | T1 Transaction Type | 43h: ‘C’ Canteen data |
| 28 | T1 Year | 0dto 99d |
| 29 | T1 Month | 1d to 12d |
| 30 | T1 Date | 1d t o31d |
| 31 | T1 Hour | 0d to 23d |
| 32 | T1 Minute | 0d to 59d |
| 33 | T1 Second | 0d to 59d |
| 34 | T1 Access Info |  |
| 35 | T1 Reader Number | 1d to 8d |
| 36 | T1 Door Number | 1d to 4d |
| 37 | T1 RFU | 0h |
| 38 | T1 In punch / out punch | 0d: null 1d: in 2d: out |
| 39 | T1 Reason Flag | 0h: no reason, any other value (see reason codes) |
| 40 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 41 | T1 Trans ID LSB |  |
| 42 | T1 Trans ID HSB |  |
| 43 | T1 Trans ID USB |  |
| 44 | T1 Trans ID MSB |  |
| 45 | T1 Item Number LSB |  |
| 46 | T1 Item Number MSB |  |
| 47 | T1 Item Quantity |  |
| 48 | T1 Item Price LSB |  |
| 49 | T1 Item Price HSB |  |
| 50 | T1 Item Price USB |  |
| 51 | T1 Item Price MSB |  |
| 42 to … | T2 data |  |
| … to … | T3 data |  |
| … to N-2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Recovery Transaction Frame (E8h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | E7h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | Unique Sequence Number LSB | 4byte unique id |
| 17 | Unique Sequence Number HSB |  |
| 18 | Unique Sequence Number USB |  |
| 19 | Unique Sequence Number MSB |  |
| 20 | No. of Transactions in this frame | 1d to 32d |
| 21 | T1 Transaction type | 5h: recovered punch transactions |
| 22 | T1 CID LSB |  |
| 23 | T1 CID HSB |  |
| 24 | T1 UID LSB |  |
| 25 | T1 UID HSB |  |
| 26 | T1 UID USB |  |
| 27 | T1 UID MSB |  |
| 28 | T1 SC LSB |  |
| 29 | T1 SC HSB |  |
| 30 | T1 SC USB |  |
| 31 | T1 SC MSB |  |
| 32 | T1 Year | 0dto 99d |
| 33 | T1 Month | 1d to 12d |
| 34 | T1 Date | 1d t o31d |
| 35 | T1 Hour | 0d to 23d |
| 36 | T1 Minute | 0d to 59d |
| 37 | T1 Second | 0d to 59d |
| 38 | T1 Access Info / Input Number | See Access Events / Input numbers |
| 39 | T1 Reader Number | 1d to 8d |
| 40 | T1 Door Number | 1d to 4d |
| 41 | T1 Area number | 1d to 4d |
| 42 | T1 In punch / out punch | 0d: in 1d: out |
| 43 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 44 | T1 Trans ID LSB |  |
| 45 | T1 Trans ID HSB |  |
| 46 | T1 Trans ID USB |  |
| 47 | T1 Trans ID MSB |  |
| 48 to --- | T2 data |  |
| --- to --- | T3 data |  |
| --- to N - 2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Recovery Report Frame (E9h)

Legend:

Missed: not uploaded, missed transaction found during recovery

Uploaded: uploaded transaction found during recovery

Garbage: useless / corrupt transaction found during recovery

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | E8h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | Unique Sequence Number LSB | 4byte unique id |
| 17 | Unique Sequence Number HSB |  |
| 18 | Unique Sequence Number USB |  |
| 19 | Unique Sequence Number MSB |  |
| 20 | Error Flag | 1h: recovery Success & finished 2h: end date not found 3h: start data not found |
| 21 | Missed LSB | DDh (if AABBCCDDh) CCh BBh AAh (for internal purpose only.. do not show this info on screen) |
| 22 | Missed HSB |  |
| 23 | Missed USB |  |
| 24 | Missed MSB |  |
| 25 | Uploaded LSB |  |
| 26 | Uploaded HSB |  |
| 27 | Uploaded USB |  |
| 28 | Uploaded MSB |  |
| 29 | Garbage LSB |  |
| 30 | Garbage HSB |  |
| 31 | Garbage USB |  |
| 32 | Garbage MSB |  |
| 33 | LRC |  |
| 34 | Footer | Fixed BBh |

## Get Pending Event list (EAh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | 0Fh |
| 2 | Total Length (HSB) | 00h |
| 3 | Command Or Response | 01h / 02h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | EAh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | 04h |
| 9 | Payload length (HSB) | 00h |
| 10 | Pending LSB | DDh (if AABBCCDDh) CCh BBh AAh (Where AABBCCDDh = no. of pending events) |
| 11 | Pending HSB |  |
| 12 | Pending USB |  |
| 13 | Pending MSB |  |
| 14 | LRC |  |
| 15 | Footer | Fixed BBh |

## New Transaction Frame (EBh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | EBh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | No. of transactions in this frame | 1d to 32d |
| 17 | Size of each Transaction | 31d / 1Fh |
| 18 | T1 UID LSB |  |
| 19 | T1 UID HSB |  |
| 20 | T1 UID USB |  |
| 21 | T1 UID MSB |  |
| 22 | T1 SC LSB |  |
| 23 | T1 SC HSB |  |
| 24 | T1 SC USB |  |
| 25 | T1 SC MSB |  |
| 26 | T1 CID LSB |  |
| 27 | T1 CID HSB |  |
| 28 | T1 Transaction Type | 50h: ‘P’ punch Transaction 48h: ‘H’ hard alarm 53h: ‘S’ soft alarm 47h: ‘G’ gps data 43h: ‘C’ Canteen data |
| 29 | T1 Year | 0dto 99d |
| 30 | T1 Month | 1d to 12d |
| 31 | T1 Date | 1d t o31d |
| 32 | T1 Hour | 0d to 23d |
| 33 | T1 Minute | 0d to 59d |
| 34 | T1 Second | 0d to 59d |
| 35 | T1 Access Info / Input Number | See Access Events / Input numbers |
| 36 | T1 Reader Number | 1d to 8d |
| 37 | T1 Door Number | 1d to 4d |
| 38 | T1 RFU | 0h |
| 39 | T1 In punch / out punch | 0d: null 1d: in 2d: out |
| 40 | T1 Reason Flag | 0h: no reason, any other value (see reason codes) |
| 41 | T1 Is Photo available | 0d: no photo captured 1d: photo captured |
| 42 | T1 Trans ID LSB |  |
| 43 | T1 Trans ID HSB |  |
| 44 | T1 Trans ID USB |  |
| 45 | T1 Trans ID MSB |  |
| 46 | T1 Temperature Integer part | XXh (if temp = XX.YY Degree Celsius) |
| 47 | T1 temperature Fraction Part | YYh |
| 48 | T1 RFU |  |
| 49 to … | T2 data |  |
| … to … | T3 data |  |
| … to N-2 | T4 to T32 data |  |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

## Push Log (ECh)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | ECh |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | MAC ID 1 | 11h (if mac id is 11:22:33:44:55:66) |
| 11 | MAC ID 2 | 22h |
| 12 | MAC ID 3 | 33h |
| 13 | MAC ID 4 | 44h |
| 14 | MAC ID 5 | 55h |
| 15 | MAC ID 6 | 66h |
| 16 | Admin UID LSB |  |
| 17 | Admin UID HSB |  |
| 18 | Admin UID USB |  |
| 19 | Admin UID MSB |  |
| 20 | Year | 0dto 99d |
| 21 | Month | 1d to 12d |
| 22 | Date | 1d t o31d |
| 23 | Hour | 0d to 23d |
| 24 | Minute | 0d to 59d |
| 25 | Second | 0d to 59d |
| 26 | RFU |  |
| 27 | RFU |  |
| 28 | RFU |  |
| 29 | RFU |  |
| 30 to N-2 | Log Data | ASCII String, max 235 bytes |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

# Query commands

## General Query to Server (D0h)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | D0h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Query Type | 01d: Check Balance / Bill |
| 11 | UID LSB | 78h(If UID is 0x12345678) |
| 12 | UID HSB | 56h |
| 13 | UID USB | 34h |
| 14 | UID MSB | 12h |
| 15 to n-2 | Null terminated response, present only in response frame from server. | Max 1023 ASCII char. Each lines must be saperated by ‘\n’. last byte must be 00h. |
| n-1 | LRC |  |
| n | Footer | Fixed BBh |

## Server Authentication Query / Response (D1h)

Query Frame (From Device)

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 00h |
| 6 | FC | D1h |
| 7 | Response Required? /Response | 00h / XXh |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Auth ID LSB | 78h (if auth id is 12345678h) |
| 11 | Auth ID HSB | 56h |
| 12 | Auth ID USB | 34h |
| 13 | Auth ID MSB | 12h |
| 14 | Rule Number LSB | YYh (where rule number is XXYYh) |
| 15 | Rule Number HSB | HHh |
| 16 | Door Number | 1h2h,3h or 4h |
| 17 | InOut | 1h: In 2h: Out 0h: Null |
| 18 | UID LSB | Card ID |
| 19 | UID HSB |  |
| 20 | UID USB |  |
| 21 | UID MSB |  |
| 22 | Extd LSB | Extended Card ID (for cards more then 32 bits) RFU as of now |
| 23 | Extd HSB |  |
| 24 | Extd USB |  |
| 25 | Extd MSB |  |
| 26 | Hour | 01d to 23d |
| 27 | Minute | 00d to 59d |
| 28 | Second | 00d to 59d |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

Response Frame (From Server )

| Byte Index | Data | Value |
| --- | --- | --- |
| 0 | Header | AAh |
| 1 | Total Length (LSB) | YYh |
| 2 | Total Length (HSB) | XXh |
| 3 | Command Or Response | 01h |
| 4 | RFU | 01h |
| 5 | R/W flag | 01h |
| 6 | FC | D1h |
| 7 | Response Required? /Response | 01h //send success always here |
| 8 | Payload length (LSB) | YYh |
| 9 | Payload length (HSB) | XXh |
| 10 | Auth ID LSB | 78h (if auth id is 12345678h) |
| 11 | Auth ID HSB | 56h |
| 12 | Auth ID USB | 34h |
| 13 | Auth ID MSB | 12h |
| 14 | Result | 00h: No Say 01h: Allowed 02h: Denied |
| 15 + 16 | Deny Reason | 16 byte ASCII encoded deny string (optional, send all zeros if not available) |
| 32 | Fixed zero | 00h |
| N - 1 | LRC |  |
| N | Footer | Fixed BBh |

# Change Log

- version 2.00 P

  - Added this section.

  - Changed 43h command

  - Added command (47h)

  - Removed definition of capture from command number (1Dh & 1Eh). now only alarm can be enabled or disabled. Capture option is moved to command number (47h)

- Version 2.00 R

  - Interlocking rule changed byte 17 from “RFU” to “No of doors”

  - Interlocking rule added bytes 66 67 68 (fail safe / fail secure mechanism)

- Version 2.00S

  - Removed scope variables from all inputs in command Input Definitions (1Ch)

  - Added scope variables in command Input Action (1Dh)

  - Added canteen commands for (C0h to C4h)

- Version 2.00T

  - Added password reset command

  - Added ble para command

- Version 2.00U

  - Modified ble parameter command

- Version 2.00V

  - Changed mifare config commands.

- Version 2.00Y

  - Changed 20h (display parameter command little bit) added card id display format in an rfu byte

- Version 2.00Z

  - Changed 2Dh command entirely..

- Version 2.01

  - Changed Display & brightness para command.

  - Changed minor version of this document

- Version 2.02

  - Command 17h (Reader setup) > byte index [19] > replaced from RFU to Transaction enable / disable?

- Version 2.03

  - Command 4Ah (Finger Encryption added).

- Version 2.04

  - Command 4Bh (Extended User Information) added.

  - Rule :  Multiple Authentication (40h)Deprecated.

  - Rule Added Group Authentication (40h) Added.

- Version 2.05

  - Command F9h (Enable / Disable Ethernet modules)added.

- Version 2.06

  - Added command 2Ch (volume para)

- Version 2.07

  - Added command 2Dh (User password/ pin reset/change)

- Version 2.08

  - Updated command 16h (Controller setup) . new parameter added, wiegand out enabled in controller mode

- Version 2.09

  - Corrected command 4Dh (user pin change), added byte for door id

- Version 2.10

  - Updated some finger print parameter values.

- Version 2.11

  - Added command EAh = Get pending event from service (previously EAh was designated for Push Canteen Data, but was not getting used, was duplicate of E7h, so removed header)

- Version 2.12

  - Added command Escort Rule setup (4Eh).

  - Modified command User Extended Info (4Bh).

  - Renamed Obsolete Bytes named Site Code (SC) in various commands to Extended ID. Commands modified are 03h,04h,07h,0Ch,0Dh,2Dh,32h,3Bh,3Dh,41h,4Dh.

- Version 2.13

  - Added command MQTT Publisher Rule setup (4Fh)

- Version 2.14

  - Modified Controller Setup command (16h / 17h) To indicate new type transaction frame

  - Added a new command Temperate Sensor Para (50h).

  - Added new push command New Type Transaction Frame (EBh).

- Version 2.16

  - Added command 51h

- Version 2.17

  - Added command D0h

- Version 2.18

  - Added command 52h, Mifare MAD Config / Paramters

- Version 2.22

  - Added command 53h, 54h related OSDP setup

  - EAC reader setup command (55h)	 code id changed from ECh

- Version 2.23

  - Amended Matrix APB rule 38H

- Version 2.25

  - Added Command D1h (Server auth query & response).

  - Added command 56h (Server auth rule setup).

- Version 2.26

  - Input to Output Trigger Command (1Dh). accommodate rule reset on GPI Trigger.

- Version 2.27

  - Added command 57h and 58h

- Version 2.28

  - Added Command, ECh (Push Log)

  - Modified Command 48h

# Memory_management

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | TZ 1 Start Hour (fixed read only) | 0 |
| 2 | TZ 1 Start Minute (fixed read only) | 0 |
| 3 | TZ 1 End Hour (fixed read only) | 23 |
| 4 | TZ 1 End Minute (fixed read only) | 59 |
| 5 to … |  | 0 to 59 |
|  |  | 0 to 59 |
|  |  | 0 to 59 |
| 504 | TZ 127 Start Hour | 0 to 59 |
| 506 | TZ 127 Start Minute | 0 to 59 |
| 507 | TZ 127 End Hour | 0 to 59 |
| 508 | TZ 127 End Hour | 0 to 59 |
| 509 | Check Sum_1st_Byte | 0 to 255 |
| 510 | Check Sum_2nd_Byte | 0 to 255 |
| 511 | Check Sum_3d_Byte | 0 to 255 |
| 512 | Check Sum_4th_Byte | 0 to 255 |

- Timezone Table

2. TimeZone Xs Group.

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | Group1_1st Time Zone for Gate 1 | 1 to 127 |
| 2 | Group1_2st Time Zone for Gate 1 | 1 to 127 |
| 3 | Group1_3st Time Zone for Gate 1 | 1 to 127 |
| 4 | Group1_4st Time Zone for Gate 1 | 1 to 127 |
| 5 | Group1_Allow / Deny for Gate 1 | 00h : allow 01h : deny |
| 6 | Group1_1st Time Zone for Gate 2 |  |
| 7 | Group1_2st Time Zone for Gate 2 |  |
| 8 | Group1_3st Time Zone for Gate 2 |  |
| 9 | Group1_4st Time Zone for Gate 2 |  |
| 10 | Group1_Allow / Deny for Gate 2 |  |
| 11 | Group1_1st Time Zone for Gate 3 |  |
| 12 | Group1_2st Time Zone for Gate 3 |  |
| 13 | Group1_3st Time Zone for Gate 3 |  |
| 14 | Group1_4st Time Zone for Gate 3 |  |
| 15 | Group1_Allow / Deny for Gate 3 |  |
| 16 | Group1_1st Time Zone for Gate 4 |  |
| 17 | Group1_2st Time Zone for Gate 4 |  |
| 18 | Group1_3st Time Zone for Gate 4 |  |
| 19 | Group1_4st Time Zone for Gate 4 |  |
| 20 | Group1_Allow / Deny for Gate 4 |  |
| 21 | Group1_Check Sum_1st_Byte_ | 0 to 255 |
| 22 | Group1_Check Sum_2nd_Byte | 0 to 255 |
| 23 | Group1_Check Sum_3d_Byte | 0 to 255 |
| 24 | Group1_Check Sum_4th_Byte | 0 to 255 |
| … |  |  |
| … |  |  |
| … |  |  |
| 24568 | Group1_1st Time Zone for Gate 4 |  |
| 24569 | Group1_2st Time Zone for Gate 4 |  |
| 24570 | Group1_3st Time Zone for Gate 4 |  |
| 24571 | Group1_4st Time Zone for Gate 4 |  |
| 24572 | Group1_Allow / Deny for Gate 4 |  |
| 24573 | Group1_Check Sum_1st_Byte |  |
| 24574 | Group1_Check Sum_2nd_Byte |  |
| 24575 | Group1_Check Sum_3d_Byte |  |
| 24576 | Group1_Check Sum_4th_Byte |  |

3.Week Of Table

| Byte No | Data | Value |
| --- | --- | --- |
| 1 | Group 1 Week 1 Days | b0XXXXXXXX 0b0SFTWTMS 1b: day on 0b: day off |
| 2 | Group 1 Week 2 Days |  |
| 3 | Group 1 Week 3 Days |  |
| 4 | Group 1 Week 4 Days |  |
| 5 to … | Group 1 Week 5 Days |  |
| 6 to … |  |  |
| 157 | Group 1 Week 1 Days |  |
| 158 | Group 1 Week 1 Days |  |
| 159 | Group 1 Week 1 Days |  |
| 160 | Group 1 Week 1 Days |  |
| 161 | Check Sum_1st_Byte | 0 to 255 |
| 162 | Check Sum_2nd_Byte | 0 to 255 |
| 163 | Check Sum_3d_Byte | 0 to 255 |
| 164 | Check Sum_4th_Byte | 0 to 255 |

4.Holiday Xs Group Table

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | Group1_Date 1 | 1 to 31 |
| 2 | Group1_Month 1 | 1 to 12 |
| 3 | Group1_Year 1 | 00 to 99 |
| 4 | Group1_Date 2 |  |
| 5 | Group1_Month 2 |  |
| 6 | Group1_Year 2 |  |
| 7 to ….. |  |  |
| 382 | Group1_Date 128 |  |
| 383 | Group1_Month 128 |  |
| 384 | Group1_Year 128 |  |
| 385 | Check Sum_1st_Byte_Group1 | 0 to 255 |
| 386 | Check Sum_2nd_Byte_Group1 | 0 to 255 |
| 387 | Check Sum_3d_Byte_Group1 | 0 to 255 |
| 388 | Check Sum_4th_Byte_Group1 | 0 to 255 |
| 389 to ….. |  |  |
| 12028 | Group32_Date 1 | 1 to 31 |
| 12029 | Group32_Month 1 | 1 to 12 |
| 12030 | Group32_Year 1 | 00 to 99 |
| 12031 | Group32_Date 2 |  |
| 12032 | Group32_Month 2 |  |
| 12033 | Group32_Year 2 |  |
| 12034 to ….. |  |  |
| 12410 | Group32_Date 128 |  |
| 12411 | Group32_Month 128 |  |
| 12412 | Group32_Year 128 |  |
| 12413 | Check Sum_1st_Byte_Group32 | 0 to 255 |
| 12414 | Check Sum_2nd_Byte_Group32 | 0 to 255 |
| 12415 | Check Sum_3d_Byte_Group32 | 0 to 255 |
| 12416 | Check Sum_4th_Byte_Group32 | 0 to 255 |

- ID Data base .

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | 1st_User_UID LSB | 0 to 255 |
| 2 | 1st_User_UID HSB | 0 to 255 |
| 3 | 1st_User_UID USB | 0 to 255 |
| 4 | 1st_User_UID MSB | 0 to 255 |
| 5 to … |  |  |
| 999997 | 2.5Lac_User_UID LSB | 0 to 255 |
| 999998 | 2.5Lac_UID HSB | 0 to 255 |
| 999999 | 2.5Lac_UID USB | 0 to 255 |
| 1000000 | 2.5Lac_UID MSB | 0 to 255 |

6.User Database

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | User_UID LSB |  |
| 2 | User_UID HSB |  |
| 3 | User_UID USB |  |
| 4 | User_UID MSB |  |
| 5 | SC _UID LSB |  |
| 6 | SC _UID HSB |  |
| 7 | SC _UID USB |  |
| 8 | SC _UID MSB |  |
| 9 | Time Zone Wise / Exclusive |  |
| 10 | User Type |  |
| 11 | Week Off group number LSB |  |
| 12 | Week Off group number HSB |  |
| 13 | Holiday Access group LSB |  |
| 14 | Holiday Access group HSB |  |
| 15 | Start Year |  |
| 16 | Start Month |  |
| 17 | Start Date |  |
| 18 | Start Hour |  |
| 19 | Start Minute |  |
| 20 | End Year |  |
| 21 | End Month |  |
| 22 | End Date |  |
| 23 | End Hour |  |
| 24 | End Minute |  |
| 25 | Pin LSB |  |
| 26 | Pin HSB |  |
| 27 | Pin USB |  |
| 28 | Pin MSB |  |
| 29 to 60 | User Name (32 Bytes) |  |
| 61 to 76 | Employee ID (16 Bytes) |  |
| 77 | Time Zone Group ID 1 LSB | (Item Xs Group) |
| 78 | Time Zone Group ID 1 HSB |  |
| 79 | Time Zone Group ID 2 LSB |  |
| 80 | Time Zone Group ID 2 HSB |  |
| 81 | Time Zone Group ID 3 LSB |  |
| 82 | Time Zone Group ID 3 HSB |  |
| 83 | Time Zone Group ID 4 LSB |  |
| 84 | Time Zone Group ID 4 HSB |  |
| 85 | Time Zone Group ID 5 LSB |  |
| 86 | Time Zone Group ID 5 HSB |  |
| 87 | Time Zone Group ID 6 LSB |  |
| 88 | Time Zone Group ID 6 HSB |  |
| 89 | Time Zone Group ID 7 LSB |  |
| 90 | Time Zone Group ID 7 HSB |  |
| 91 | Time Zone Group ID 8 LSB |  |
| 92 | Time Zone Group ID 8 HSB |  |
| 93 to 96 | Escort ID (4 Bytes) |  |
| 97 to 100 | Escort Site Code (4 Bytes) |  |

- Fingerprint Data base :

| Sector No. | Data |
| --- | --- |
| 1 | 1st_User_UID Fingerprint_1 (384 Bytes) |
| 2 | 1st_User_UID Fingerprint_2 (384 Bytes) |
| 3 | 1st_User_UID Fingerprint_3 (384 Bytes) |
| 4 | 1st_User_UID Fingerprint_4 (384 Bytes) |
| 5 | 1st_User_UID Fingerprint_5 (384 Bytes) |
| 6 | 1st_User_UID Fingerprint_6 (384 Bytes) |
| 7 | 1st_User_UID Fingerprint_7 (384 Bytes) |
| 8 | 1st_User_UID Fingerprint_8 (384 Bytes) |
| 9 | 1st_User_UID Fingerprint_9 (384 Bytes) |
| 10 | 1st_User_UID Fingerprint_10 (384 Bytes) |
| 11 | 1st_User_Finger_Registration_Info (10 Bytes) |
| … |  |
| … |  |
| 27,49,990 | 2.5st_User_UID Fingerprint_1 (384 Bytes) |
| 27,49,991 | 2.5st_User_UID Fingerprint_2 (384 Bytes) |
| 27,49,992 | 2.5st_User_UID Fingerprint_3 (384 Bytes) |
| 27,49,993 | 2.5st_User_UID Fingerprint_4 (384 Bytes) |
| 27,49,994 | 2.5st_User_UID Fingerprint_5 (384 Bytes) |
| 27,49,995 | 2.5st_User_UID Fingerprint_6 (384 Bytes) |
| 27,49,996 | 2.5st_User_UID Fingerprint_7 (384 Bytes) |
| 27,49,997 | 2.5st_User_UID Fingerprint_8 (384 Bytes) |
| 27,49,998 | 2.5st_User_UID Fingerprint_9 (384 Bytes) |
| 27,49,999 | 2.5st_User_UID Fingerprint_10 (384 Bytes) |
| 27,50,000 | 2.5st_User_Finger_Registration_Info (10 Bytes) |

7. Item Database :

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | Item Number LSB |  |
| 2 | Item Number MSB |  |
| 3 | Active / Inactive |  |
| 4 | Item Price (LSB) |  |
| 5 | Item Price (HSB) |  |
| 6 | Item Price (USB) |  |
| 7 | Item Price (MSB) |  |
| 8 | Item Name Byte1 |  |
| … | … |  |
| 23 | Item Name Byte16 |  |
| 24 | Week Day Group |  |
| 25 | Canteen Code |  |
| 26 | Vendor Code |  |

8. Item Group :

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1 | Location ID |  |
| 2 | Item Group Number |  |
| 3 | Prepaid / PostPaid |  |
| 4 | Item Number LSB |  |
| 5 | Item Number MSB |  |
| 6 | Timezone1 Number |  |
| 7 | Timezone2 Number |  |
| 8 | Timezone3 Number |  |
| 9 | Timezone4 Number |  |
| 10 | Item Restriction Type |  |
| 11 | Item Restriction Quantity |  |
| 12 | Subsidize price LSB |  |
| 13 | Subsidize price MSB |  |
| 14 | Subsidize price USB |  |
| 15 | Subsidize price HSB |  |
| 16 | Subsidize Restriction Type |  |
| 17 | Subsidize Restriction Quantity |  |

9. Vendor Database:

| Byte No | Data | Value(d) |
| --- | --- | --- |
| 1... | Vendor1 Name Byte1 |  |
| …32 | Vendor1 Name Byte32 |  |
| 33… | Vendor2 Name Byte1 |  |
| …64 | Vendor2 Name Byte32 |  |
| … |  |  |
| … |  |  |
| … |  |  |
| 4064… | Vendor128 Name Byte1 |  |
| …4096 | Vendor128 Name Byte32 |  |